Reference Guide

RAIL / ROAD INTERFACE

V 1.3 (August 2025)

Written by: Philippe Cavenel

17 July 2023 
Last update August, 27 2025

Documentation Version 1.10

# Table of contents

Table of contents	1

Illustrations	4

1	Preamble	5

1.1	History	5

1.2	Abbreviations	5

1.3	Typographic convention	5

2	Disclaimer	6

3	Introduction	7

3.1	Document purpose	7

Why this document?	7

Document content	7

Warning	7

3.2	Technical References	8

Eagle 6.6.0	8

POV-Ray 3.7	8

MPLAB 8.92	9

PICKIT 2	9

4	Electronic design	11

4.1	Global architecture	11

4.2	Data transfer between boards	12

4.3	Data transfer between master board and PC	13

4.4	Schematic	14

4.5	Pin assignment	19

4.6	Place and route	20

Top view	20

Bottom view	20

Components view	21

3D view	22

4.7	BOM	23

4.8	Generate the manufacturing files	24

First step: ERC on Eagle	24

Step two: DRC and silkscreen on Eagle	24

Step three: CAM processor on Eagle	24

Fourth stage: Final rendering	24

How to add a 3d model	25

Fifth stage: BOM	26

Sixth step: send files for manufacturing	26

5	Firmware design	27

5.1	Set PIC18F frequency to 32MHz	27

5.2	CAN bus control library	27

Introduction	27

Module Features	27

List of Component Modules	27

Functions	28

CAN Initialization	28

CAN Reception	28

5.3	PWM management in analog mode, DCC management in NMRA digital mode and TM1637 device management	32

5.4	Flash storage of programming information	40

5.5	Automation management	44

5.6	Software design	45

5.7	Protocol on serial link	45

5.8	BNF Grammar	45

5.9	Protocol on CAN bus	46

5.10	Output display and knob controls	48

6	Command	49

Programming DCC mode on a board	49

Programming ANA mode on a board	50

Programming AUTOMATIC mode on a board	51

Programming MANUAL mode on a board	52

Initialize a GPIO as output on a board	53

Initialize a GPIO as input on a board	54

Program an automatic action on a GPIO driven by a timer	55

Program an automatic action on a LPO driven by a timer	56

Program an automatic action on a track driven by a timer	57

Program on a timer driven by a timer	58

Turn On or Off an automation driven by a timer	59

Program an automatic action on a GPIO driven by a change of level on a GPIO	60

Program an automatic action on a LPO driven by a change of level on a GPIO	61

Program an automatic action on a track driven by a level change on a GPIO	62

Program an automatic action on a loco (DCC) driven by a level change on a GPIO	63

Program on a timer driven by a level change on a GPIO	64

Turn on or off an automation by a level change on a GPIO	65

Program an automatic action on a GPIO driven by a change of vehicle presence on a track	66

Program an automatic action on a LPO driven by a change of vehicle presence on a track	67

Program an automatic action on a track driven by a change of vehicle presence on a track	68

Program an automatic action on a loco (DCC) by a change of vehicle presence on a track	69

Program on a timer driven by a change of vehicle presence on a track	70

Turn on or off an automation by a change of vehicle presence on a track	71

Delete an automation on a board	72

Force the value on a GPIO on a board	73

Create a timer on a board	74

Force the value on a LPO on a board	75

Turn on an automation on a board	76

Turn off an automation on a board	77

Force speed and direction on a track	78

Send a DCC command to a board	79

Request the status of all the GPIOs on a board	80

Request the status of all the LPOs on a board	81

Request track status on a board	82

Request board status	83

Request the list of actions programmed on a board	84

Request a dump of memory on a board	85

Request a calibration of knobs	86

6.1	Global command	87

Stop all	87

Run all	87

Run a specific board	87

Reset all automation of a specific board	87

Inconsistent programming	88

7	Barraux's railway network (31/07/2025)	89

General System Structure	89

⚙️ Operating Modes	89

🧠 Automation per Board	90

📌 TCO (Optical Control Panel) Commands	91

📌 Programing the boards	91

Source Code and Project Access	92

Board Code (27/08/2025)	94

8	Plants & Hardware Systems	104

8.1	"Entry-level" ≲ 300 €	104

8.2	"Mid-range" ~ €300–600	104

8.3	"High-end / club / large network" ≥ 600 €	105

8.4	PC-centric solutions & software (for automation)	105

8.5	DIY/open-source (very economical)	105

9	Market Estimate (units/year)	105

9.1	Quick advice to choose	106

🔎 Key features of this DIY interface	106

💰 Positioning in relation to the existing market	107

📊 Likely range of commercialization	107

🚀 Market potential	108

10	🛠️ Product Range Simulation	108

10.1	"Starter Master" Pack	108

10.2	"Slave Expansion" card	108

10.3	Advanced Automation Pack	109

10.4	"AMS Road Integration" package	109

11	📊 Summary range	109

12	🚀 Market strategy	109

13	📑 Mini Business Plan – Rail Interface/AMS	110

13.1	Product	110

13.2	Market and potential	110

13.3	Revenue model (per 1,000 customers)	110

13.4	Cost structure	111

13.5	Fixed Expenses (Year 1)	111

13.6	Estimated ROI	111

13.7	Competitive comparison (price per controlled section)	111

14	🎯 Conclusion	112

# Illustrations

Figure 1 PICkit 2 Programmer Connector pinout	10

Figure 2 PWM Mode	11

Figure 3 DCC NMRA mode	11

Figure 4 Master/slave board operating diagram	12

Figure 5 CAN bus wiring diagram	13

Figure 6 Transceiver MCP2551 wiring diagram	13

Figure 7 (Proposed by Kyle Thomson Revised 8/18/2009)	14

Figure 8 5V power supply with standard 7805	14

Figure 9 CAN and RS232 interface	15

Figure 10 Master or Slave board	15

Figure 11Power boost	16

Figure 12 Vehicle Presence Detection	16

Figure 13 Top, Pads, Vias	20

Figure 14 Bottom, Pads, Vias	20

Figure 15 Pads, Vias, tPlace, tValues	21

Figure 16 Top View (800 x 600)	22

Figure 17 Bottom View (800 x 600)	22

Figure 18 Led Display on TCO	48

Figure 19 Railway network (31/07/2025)	89

Figure 20 Rail Manager UI, sribe interface	92

Figure 21 Rail Manager UI, node interface	93

Figure 22 Rail Manager UI, node highlighted	93

# Preamble

## History

| History | Exchange | Date |
| --- | --- | --- |
| Version 1.0 | Creation | 18/May/2021 |
| Version 1.1 | New PCB design | 18/May/2023 |
| Version 1.2 | Replace H-bridge transistors with L298s | 22/July/2023 |
| Version 1.3 | Add more information on last development with PIC18F4680 | 30/07/2025 |

## Abbreviations

DCC: 		Digital Command Control (defined by a NMRA standard)

NMRA: 	National Model Railroad Association

AMS: 		Auto Motor Sport (Faller)

HO: 		Half-O (model railway scale corresponding to 1:87)

DIY: 		Do It Yourself

PC: 		Personal Computer

USB: 		Universal Serial Bus

FTDI:		 Future Technology Devices International

CAN (bus): 	Controller Area Network

TTL: 		Transistor-Transistor logic

GPI : 		Global Purpose Input-Output

LED: 		Light-Emitting Diode

PWM: 	Pulse Width Modulation

EMC: 		ElectroMagnetic Compatibility

STL:		STereo-Lithography

STEP:		STandard for the Exchange of Product model data

DRC:		Design Rule Check

ERC:		Electrical Rule Check

## Typographic convention

The file names are in Courier New format, for example RailDriverCamJob.cam file

The online order examples or file contents are in bold Courier New on a grey background, for example

stl2pov.exe myModel.stl > myModel.inc.

Warning is in bold red inside a frame

Binary names are in bolt like for example stl2pov.exe

# Disclaimer

The information contained in this document has been obtained from sources believed to be reliable. However, it may contain technical inaccuracies or typographical errors.

The author reserves the right to correct such errors as soon as they are brought to his attention. We strongly recommend that you check the accuracy and relevance of the information provided in this document.

The information contained in this document is subject to change at any time, and may have been updated. In particular, it may have been updated between the time it was downloaded and the time the user takes cognizance of it.

Use of the information contained in this document is at the user's sole risk, and the user assumes full responsibility for any consequences arising therefrom, without any liability or recourse against the author.

In no event shall the author be liable for any damages whatsoever resulting from the interpretation or use of the information contained herein.

# Introduction

## Document purpose

### Why this document?

Faller AMS car networks do not have a DCC-type control mode (NMRA standard). This makes it difficult to have several vehicles on the same network. Using diodes, it is possible to operate a maximum of 2 vehicles per track, which is not very many. By creating track sections and vehicle detections per track section, you can have many more vehicles on the track. On the other hand, the realism of these networks is not respected with operating speeds that are too high, and by using PWM-type pulsed current mechanism, a much more realistic effect can be achieved.

As soon as this type of board is developed, it seems interesting to have a universal programmable interface that can also be used to drive a railway network, and thus support DCC mode (NMRA).

### Document content

This document describes the development of a DIY Faller AMS network board for HO rail and road models and the various associated software (Firmware for PIC18F4585 and PIC18F4680 and PC software) developed since 2021.

The PIC18F4680 allows for storing more program data in memory; the only difference in firmware programming is the use of the correct 18f4680_g.lkr file during compilation.  The use of this microcontroller occurred during the course of the project development, so it is possible that other models may still be referenced in this document. However, the code available on GitHub has been properly developed for the 4680 version of the PIC18F.

The board is designed to communicate with a PC via an RS232 serial link (or via a USB connection using an FTDI cable connected to the RS232 connector, available at Amazon: DSD TECH SH-RS232G USB to DB9 Female serial cable Integrated FTDI FT232RL chip) and with each other via a CAN bus.

Each board can drive up to 4 sections in analog or digital DCC mode (NMRA standard) with electric vehicle presence detection, and 6 low-power controls for lighting or small mechanisms (such as turnout controls). Finally, 8 TTL inputs/outputs (GPIOs) 4 are dedicated to vehicle presence detection, and 4 enable the addition of detectors, control LEDs, or the linking of boards to generate automatic control mechanisms (blocking).

Boards cannot individually mix DCC and analog modes, but it is possible to run one board in DCC mode and another in analog mode.

Each board has a 5-bit address (so 31 can be used, i.e. 128 slots for the whole network). The PC interface board called the "master" is set to address 31 by default. Each other boards should have at least the bit 0 or 1 or 2 set to 0.

### Warning

The current 2023 version at the time of writing is V1.2, and this document describes this version.

This board was developed for personal usage and on an old but readily available and quickly mastered environment that can simply be replaced by a tool like KiCad EDA or Altium on more recent computers.

All technical indications (such as layer numbers in the Eagle tool) therefore refer to these tools (see Technical References below). To simplify use of this documentation, a zip file containing all developments is available for download.

Finally, the development of a professional version would require the use of SMD components, a minimum EMC and countries compliance study, integration into a great casing, cost analysis, user manual, and targeting of customers for a possible market launch. This is not the purpose of this documentation. This development should be considered as a simple prototype for personal usage.

## Technical References

In order to reduce development costs and simplify board assembly, the entire board is mounted using radial components, and the design was conceived on an available professional version 6.6.0 of Eagle, MPLAB 8.92 and POV-Ray 3.7 running Mac OS High Sierra on a MacBook Pro in early 2011, 2.7 GHz Intel Core I7 with 16 GB DDR3 1333 MHz memory. This computer runs Mac OS and Windows on Parallel desktop V16.5.1. Development environment needs tools on both operating system, Mac OS and Windows. Pending the development of a bootloader for the master and slave boards, the firmware is updated using a PICkit™ 2 programmer/debugger (PG164120).

### Eagle 6.6.0

EAGLE contains a schematic editor, for designing circuit diagrams. Schematics are stored in files with . SCH extension, parts are defined in device libraries with . LBR extension. Parts can be placed on many sheets and connected together through ports.

The PCB layout editor stores board files with the extension . BRD. It allows back-annotation to the schematic and auto-routing to automatically connect traces based on the connections defined in the schematic.

EAGLE saves Gerber and PostScript layout files as well as Excellon and Sieb & Meyer drill files. These are standard file formats accepted by PCB fabrication companies, but given EAGLE's typical user base of small design firms and hobbyists, many PCB fabricators and assembly shops also accept EAGLE board files (with extension . BRD) directly to export optimized production files and pick-and-place data themselves.

EAGLE provides a multi-window graphical user interface and menu system for editing, project management and to customize the interface and design parameters. The system can be controlled via mouse, keyboard hotkeys or by entering specific commands at an embedded command line. Keyboard hotkeys can be user defined. Multiple repeating commands can be combined into script files (with file extension . SCR). It is also possible to explore design files utilizing an EAGLE-specific object-oriented programming language (with extension . ULP).

(source Wikipedia)

### POV-Ray 3.7

POV-Ray (Persistence of Vision Raytracer), or POV, is a free raytracing software available on a wide variety of platforms (Windows, Mac OS, GNU/Linux, etc.). It was originally based on DKBTrace sources, and to a lesser extent on Polyray.

POV-Ray does not have an integrated graphical interface (3D modeler) like most current synthesis software, but uses scene description scripts, in which all objects, lights, etc. must be described.

Modelers dedicated solely to POV-Ray exist (KPovModeler, Moray, Yet another POV-Ray modeller...), while many others export to the POV-Ray file format. Its file format is ASCII, and the default file extension is ".pov".

This provides basic shapes (spheres, boxes, toroids, etc.) on which Boolean operations can be performed using CSG. It also makes it possible to create volumes or surfaces based on mathematical functions, such as isosurfaces.

Example: function {x*x - F/y*y + z*z} draws a kind of gravity well, with F representing its force.

It is also possible to import objects from other software (such as 3D Studio Max, Poser, etc.), which will be rendered in POV-Ray as an assembly of triangles, as many software programs are compatible, but it is very difficult to export POV-Ray objects to other formats.

POV-Ray can also be used to create animations.

(source Wikipedia)

### MPLAB 8.92

MPLAB is a proprietary freeware integrated development environment for the development of embedded applications on PIC microcontrollers, and is developed by Microchip Technology.

MPLAB X is the latest edition of MPLAB, and is developed on the NetBeans platform. MPLAB and MPLAB X support project management, code editing, debugging and programming of Microchip 8-bit PIC and AVR (including ATMEGA) microcontrollers, 16-bit PIC24 microcontrollers, as well as 32-bit SAM (ARM) and PIC32 (MIPS) microcontrollers.

MPLAB is designed to work with MPLAB-certified devices such as the MPLAB ICD 3 and MPLAB REAL ICE, for programming and debugging PIC microcontrollers using a personal computer. PICKit programmers are also supported by MPLAB.

MPLAB X supports automatic code generation with the MPLAB Code Configurator and the MPLAB Harmony Configurator plugins.

(source Wikipedia)

### PICKIT 2

The PICkit™ 2 programmer/debugger (PG164120) is a low-cost development tool with a comprehensive interface for programming and debugging microchips or microcontrollers.

Its comprehensive Windows® programming interface supports basic families (PIC10F, PIC12F5xx, PIC16F5xx), mid-range families (PIC12F6xx, PIC16F), controller families PIC18F, PIC24, dsPIC30, dsPIC33, and PIC32, 8-bit, 16-bit and 32-bit, and a large number of EEPROM-type serial microchips.

With the powerful Integrated Development Environment (IDE), PICkit™ 2 lets you debug on-circuit directly, on most PIC® microcontrollers. On-circuit debugging runs, pauses, and executes the program step by step, while the microcontroller is integrated into the application. When paused on a breakpoint, file registers can be examined and modified.

(source https://pickit2.software.informer.com  )

Figure 1 PICkit 2 Programmer Connector pinout

# Electronic design

## Global architecture

The complete system consists of a PC running, a rail or road network control application. This may be a fully developed application, or an existing one with a programmable interface driver. The interface between this PC and the control boards is via a 115200 Baud serial bus connected to a board with an RS232 serial interface. Only one of these boards, known as the "Master" board, has this interface.

The master board communicates with the other boards, known as slaves, via a 500K Baud CAN bus. The master board contains an RS232 transmitter (MAX232). By default, this board has address 31 (the address selector on this board must be removed, as well as resistors R23 and R24, giving it the value 31 by hardware setting). Address 31 is therefore forbidden on other boards, as it indicates to the PIC18F the presence of an RS232 transmitter.

If two cards have the same address, the overall behavior of the system may be inconsistent and render the entire system non-operational.

Each board is supplied with a power supply between 12 and 24V DC. The optimum supply voltage depends on the type of equipment operating on the network. The maximum theoretical voltage supported is 32V, but such a voltage could destroy the equipment connected to the board. In practice, we advise you not to exceed a supply voltage of 20V, especially if you use DCC environment.

Each board can drive 4 PWM or DCC blocks and 6 low-power modules. The boards also feature 8 TTL-level inputs/outputs (not to be used for direct control of a power module).

Figure 2 PWM Mode

Figure 3 DCC NMRA mode

The system is controlled using a command language sent to the master board via the RS232 serial link. This language has two modes: a programming mode and a control mode.

- In programming mode, it is possible to specify the direction of each board's GPIOS, and to trigger automatic actions on events, e.g. "on detection of a vehicle on block 3 of board 8, feed block 6 of board 5 in the forward direction at speed 10" (speeds are between 0 and 15).

- In control mode, commands can be given, e.g. "feed block 3 of board 2 in the reverse direction at speed 2".

Each event (detection or loss of presence of a vehicle or change of status on a GPIO or TIMER triggered) generates an event which is sent to the other boards on CAN bus.

The commands and language are described in the following chapters.

Figure 4 Master/slave board operating diagram

## Data transfer between boards

Data transfer between boards is via a 500 KBaud CAN bus. The boards are connected to the CAN bus.

|  | The first and last boards in this network have a switch for connecting a terminating resistor. |
| --- | --- |

Figure 5 CAN bus wiring diagram

A transceiver MCP2551 is used to link the PIC18F to the CAN bus.

Figure 6 Transceiver MCP2551 wiring diagram

## Data transfer between master board and PC

Data transfer between the master board and the PC is via an RS232 serial bus at 115200 baud. The link to the PC and the PIC18F is done via a MAX232 transmitter, which can be connected to an FTDI RS232/USB cable if the user so wishes.

Figure 7 (Proposed by Kyle Thomson Revised 8/18/2009)

## Schematic

The schematic consists of two parts:

- The first contains the 5V power supply (Note the STTH5L06RL inversion protection diode on this power supply). The L7805 could be replaced by a DollaTek 5V 1A regulated board, more robust. (https://www.amazon.fr/DollaTek-r%C3%A9gulateur-bornes-lentr%C3%A9e-LM7805/dp/B081JMJZG6)

Figure 8 5V power supply with standard 7805

- The RS232 and CAN bus transmitters and transceivers, the programming connector for the PIC18F and the board address selector.

Figure 9 CAN and RS232 interface

- This assembly enables the master and slave boards to be wired on the same design. For the master board, solder the RS232 DB9 connector, the MAX232 transmitter and remove resistors R23 and R24 as well as the address selector. For the slave board, do not solder the RS232 DB9 connector, nor the MAX232 transmitter but keep C14.

Figure 10 Master or Slave board

- The second contains the L298 power drivers and the 6 TIP31C transistors in Darlington circuit controlled by the PIC18F, as well as the power outputs to the rail or road network.

Figure 11Power boost

- Note that it is essential to place decoupling capacitors as close as possible to the components on the 5V supply, otherwise there is a risk of catastrophic loss of control due to PIC18F shutdown. These decoupling capacitors are 100nF on the 5V supply and 2500pF on the power outputs of the tracks to be supplied.

- Vehicle presence is detected by measuring the voltage across a diode at the output of the L298. When no load is present, a voltage close to 0V is measured at the terminal of a 2K2 resistor connected to ground, whereas when a load is present, the connection to ground is made via the 1N4004 diode, whose 0.7V bias voltage is then detected by the PIC18F.

Figure 12 Vehicle Presence Detection

## Pin assignment

## Place and route

Components are placed as close as possible to each other to limit the distance between related components, especially for decoupling capabilities. Routing is performed automatically.

### Top view

Figure 13 Top, Pads, Vias

### Bottom view

Figure 14 Bottom, Pads, Vias

### Components view

Figure 15 Pads, Vias, tPlace, tValues

### 3D view

Figure 16 Top View (800 x 600)

Figure 17 Bottom View (800 x 600)

## BOM

| Qty | VALUE | DESCRIPTION | Farnell |
| --- | --- | --- | --- |
| 1 | Standard D-Sub connectors AMPL PLUG HD20, R/A 9P, B/L,4-40 INS | X1 | 2857982 |
| 8 | 100nF | C5, C8, C9, C10, C11, C12, C13, C14 | 2309064 |
| 1 | MAX 232N | IC1 | 3121260 |
| 1 | 2000uF | C6 | 2766923 |
| 1 | 10R | R21 | 2329993 |
| 1 | 120R | R39 | 2329862 |
| 16 | 1N4148 | D3, D4, D5, D6, D7, D8, D9, D10, D12, D13, D14, D15, D17, D18, D19, D20 | 9843680 |
| 5 | 1N4004 | D22, D1, D11, D2, D16 | 1843708 |
| 1 | STTH5L06RL | D21 | 2353682 |
| 23 | 2.2K | R1, R2, R3, R4, R5, R6, R7, R8, R9, R10, R15, R20, R22, R23, R24, R25, R26, R28, R30, R32, R34, R36, R38 | 9341536 |
| 5 | 2500pF | C1, C2, C3, C4, C7 | 2860175 |
| 6 | 2N1711 | Q9, Q10, Q11, Q12, Q13, Q14 | 1611558 |
| 6 | 470 | R27, R29, R31, R33, R35, R37 | 3496822 |
| 1 | DIP / SIP Switch, 5 Circuit(s), Slide, Through Hole, SPST, 24 VDC, 100 mA | ADDRESS | 3397711 |
| 1 | Terminal block x2 | CAN | 1777096 |
| 1 | Terminal block x2 | POWER | 1777096 |
| 1 | DIP / SIP switch, 1 circuit | CAN TERMINATOR | 1960919 |
| 1 | Terminal block x12 | LOW_POWER_OUTPUT | 1777102 |
| 1 | Terminal block x8 | CONNECT_RAILS | 1777101 |
| 2 | Terminal block x4 | TRACK_STATUS, GPIO_IN(OUT) | 1777098 |
| 1 | L7805 or DollaTek 5V 1A | U3 | 1467758 |
| 1 | MCP2551P | U2 | 1439745 |
| 1 | PIC18F4585-I (or PIC18F4680) | U1 | 1439547 |
| 1 | PICKIT2 | PICKIT2 | 1187827 |
| 6 | TIP31C | T17, T18, T19, T20, T21, T22 | 9804145 |
| 2 | L298N | IC2, IC3 | 403295 |
| 2 | FK222 | KK1, KK3 | 4621281 |
| 1 | 274-1AB | KK2 | 1611445 |

## Generate the manufacturing files

### First step: ERC on Eagle

- Generate schematic and validate circuit with ERC

- Pay particular attention to net class, especially for the power section (larger surface area required)

### Step two: DRC and silkscreen on Eagle

- Component placement and automatic routing.

- To fill the top and bottom polygons, draw a polygon around the map, selecting the right layer (Top or Bottom) in the top left-hand corner after routing (otherwise automatic routing won't work), then restart routing.

- Validate with a DRC, selecting only the Top, Bottom, Via and Pad layers.

- Finalize silkscreen (layer 21) by placing all indications correctly, then generate drill legend on layer 144 by launching the ULP icon in the layout screen of Eagle on drillegend-stack.ulp

### Step three: CAM processor on Eagle

- Generate the files for CAM Processor production by clicking on the CAM_JOB/RailDriverCamJob.cam file.

- In the top-left File menu, select the .brd file for the corresponding design.

- The directories and file names must be correct in the various tabs (check and correct if necessary).

- All that's left to do is run PROCESS JOB to produce the files.

### Fourth stage: Final rendering

- To modify the selection of unknown boxes, modify the 3D_RENDERING/eagle3d/ulp/3dusrpac.dat file.

- Update the configuration files directory in 3dconf.dat

- Generate a 3D rendering of the design by launching the ULP icon in the layout screen of Eagle on 3D_RENDERING/eagle3d/ulp/3d50.ulp

- Specify the .brd file, which will generate a .pov file that can be processed by POV-Ray to obtain the 3D rendering.

- Select "User-defined model" in the General tab

- Add layer 25 in Miscellaneous Case Design

- Modify Writing on plate to display layers 21 and 25 only.

- Add layers 25 and 27 in Case Reference

- Click on Create POVRay file and Exit.

- Copy the generated EAGLE_FILES/RailDriver/RailDriver.pov file into 3D_RENDERING/eagle3d/povray

- Repeat the same operation, moving the camera to shoot from below (using Y) and/or from the other sides (using X and Z).

- Click on the 3D_RENDERING/eagle3d/povray/RailDriver.pov file to start building the 3D view with POV-Ray (automatic launch on Windows). Use 800 x 600 AA 0.3 for a reasonable image creation time, or 1600 x 1200 A 0.3 for a better resolution, but the image creation time will take several tens of minutes (On old PC).

- In Eagle, make a screen copy of the various layers built during the construction of the manufacturing files. Place all views in a PNGVIEW directory

### How to add a 3d model

- Retrieve the model in STL format from the net (snapeda for example on https://www.snapeda.com/ ). If the model is in STEP format, it must first be converted to STL, you can do that for example at on https://polyd.com/fr/convertir-step-en-stl-en-ligne

- Use the stl2pov.exe utility (launch a Windows shell)

stl2pov.exe myModel.stl > myModel.inc.

- Copy it to 3D_RENDERING/eagle3d/povray. For example:

stl2pov.exe "TO220HeatSink\TO220_Heatsink.stp">"TO220HeatSink\TO220HeatSink.inc"

- Modify the myModel.inc file so that it is understood by POV-Ray, taking as an example fk_222_sa.inc

- Add the following include to the 3D_RENDERING/eagle3d/povray/e3d_user.inc file:

#include "myModel.inc".

- Add the following information to the 3D_RENDERING/eagle3d/ulp/3dusrpac.dat file:

MYMODEL:0:1:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:MYMODEL_GRND 0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:0:

| [00] Eagle component package name [01] Output name [02] Output value [03] Define color bands [04] SMD offset (parts will be moved pcb_cuhight up/down) [05] LED options (LED options dialog box will be displayed) [06] Ready for sockets (see explanation) [07] Quartz height request [08] A part of a macro (e.g. SMD jumpers) [09] SMD resistor, generates a combination of numbers [10] Macro socket [11] Socket height in 1/10mm [12] Comments on socket [13] Internal for administration (not currently used) | [14] Y-axis rotation correction [15] Offset correction x [16] Offset correction y [17] Offset z correction [18] Use Prefix from Part? [19] Shunt on pin header (a dialog box will be displayed) [20] Logo selection dialog box is displayed [21] Reserved [29] Minimum encircling box [30] Maximum bounding box [31] POV-Ray macro (Name of pov macro and left parenthesis) [32] Package comments (German) [33] Package comments |
| --- | --- |

### Fifth stage: BOM

- Update the BOM to purchase components on Farnell or Mouser or other dealers on the net.

### Sixth step: send files for manufacturing

- Compress CAM_JOB and PNGVIEW directories to start manufacturing on a PCB manufacturing site such as AllPcb: https://www.allpcb.com/ or other manufacturer on the net.

# Firmware design

## Set PIC18F frequency to 32MHz

OSCCON = 0x70;        no pre-divider => 8MHz // comment to set 8MHz

OSCTUNE = 0x40;       activate PLL *4 => 32MHz // comment to set 8MHz

## CAN bus control library

### Introduction

This document describes programming interface to ECAN (Enhanced Controller Area Network) module.  At the time of writing this document, only PIC18F family of microcontroller contained ECAN module.  This module provides access to ECAN module in polling fashion. This module is completely written in 'C' language.  It provides many customization options that may result in significant code reduction.  To utilize this module, one must understand all options offered by ECAN module.  This module is also available in Microchip Application Note AN878.

(source ECANPoll.ReadMe file)

### Module Features

- Out-of-box support for Microchip C18 and HI-TECH PICC-18TM C compilers

- Offers simple abstract interface to ECAN module for most applications

- Additional functions/macros are available for advanced applications

- Supports all three functional modes

- Provides access to all ECAN features in Polling mode

- Easily modifiable to Interrupt-driven mode

- Operates in two main modes:

- Run-time Library Mode and Fixed Library Mode

- Various compile-time options to customization routines to a specific application

- Also available as Microchip Application Note AN878

(source ECANPoll.ReadMe file)

### List of Component Modules

| ECAN.ex.txt | This is main test file developed to demonstrate use of the library functions. |
| --- | --- |
| ECANPoll.c | This is ECAN code implementation file. One needs to include this file in their project. |
| ECANPoll.h | This file contains prototypes of functions and macros. One needs to include this file in every source file where ECAN functions will be called. |
| ECANPoll.def | This file contains all compile-time options for ECAN module. If you are using Maestro, this file will be created as per your option selections. This file is automatically included by ECANPoll.h file. |

### Functions

Refer to the ECANPoll.ReadMe file

### CAN Initialization

In order to improve performance, data reception is interrupted.

// ECAN

TRISBbits.TRISB3 = 1; // CANRX input setting

gl_outputCANbufferCounter=0;

for(bufferNumber=0;bufferNumber<MAXINPUTCANBUFFER;bufferNumber++) {

gl_inputCANWriteBufferPointer[bufferNumber]=0;

gl_inputCANReadBufferPointer[bufferNumber]=0;

gl_inputCANmode[bufferNumber]=CAN_FREE;

}

ECANInitialize(); // init ECAN

PIE3bits.RXB0IE=1; // enable interrupt for CAN

PIE3bits.RXB1IE=1; // enable interrupt for CAN

### CAN Reception

Can reception is shared with RS232 reception:

/* ==============================================================================

* Function: high_isr

* Returns: void = ISR, no return.

* Description: Hight priority interrupt service routine: receives CAN messages,

* sync handling, buffers data, and UART RX.

* ============================================================================== */

void high_isr(void){

BYTE dataLen; 				// Number of bytes transmitted in the message

BYTE dataCounter;

ECAN_RX_MSG_FLAGS flags; 	// Flags

unsigned long 	id;			// Id of sender

char	dataBuffered;

char	bufferNumber;

dataBuffered=FALSE;

if(PIR3bits.RXB0IF ||PIR3bits.RXB1IF) {

while(ECANReceiveMessage((unsigned long *)&id,(BYTE *)&gl_data[0], (BYTE *) &dataLen,(ECAN_RX_MSG_FLAGS *) &flags));

// BOARD SYNC

if (id == SYNC_ID) {

INTCONbits.TMR0IF       = 0;   // T0 int flag bit cleared before starting

TMR0H					= 0;   // re-phase Timer0

TMR0L 					= 0;   // re-phase Timer0

gl_speedCounter 		= 1;   // Same value on each board

gl_syncRequested 		= 1;   // For printing SYNC message on LED

}

// OTHER TRAMES

else {

for(bufferNumber=0;bufferNumber<MAXINPUTCANBUFFER;bufferNumber++) {

if ((unsigned long)gl_currentCANid[bufferNumber]==(unsigned long)id && (char)gl_inputCANmode[bufferNumber]!=(char)CAN_FREE) {

for(dataCounter=0;dataCounter<dataLen;dataCounter++) {

gl_inputCANbuffer[bufferNumber][gl_inputCANWriteBufferPointer[bufferNumber]]=gl_data[dataCounter];

gl_inputCANWriteBufferPointer[bufferNumber]++;

if(gl_inputCANWriteBufferPointer[bufferNumber]>=MAXTRAMESIZE)gl_inputCANWriteBufferPointer[bufferNumber]-=MAXTRAMESIZE;

}

dataBuffered=TRUE;

break;

}

}

if ((char)dataBuffered==(char)FALSE) {

for(bufferNumber=0;bufferNumber<MAXINPUTCANBUFFER;bufferNumber++) {

if (gl_inputCANmode[bufferNumber]==CAN_FREE) {

for(dataCounter=0;dataCounter<dataLen;dataCounter++) {

gl_inputCANbuffer[bufferNumber][gl_inputCANWriteBufferPointer[bufferNumber]]=gl_data[dataCounter];

gl_inputCANWriteBufferPointer[bufferNumber]++;

if(gl_inputCANWriteBufferPointer[bufferNumber]>=MAXTRAMESIZE)gl_inputCANWriteBufferPointer[bufferNumber]-=MAXTRAMESIZE;

}

gl_inputCANmode[bufferNumber]=CAN_GET_DATA;

gl_currentCANid[bufferNumber]=id;

dataBuffered=TRUE;

break;

}

}

}

}

}

if ((char)gl_master==(char)FALSE)return;

// Check if interrupt originates from USART reception

if (PIR1bits.RCIF)

{

// Read received data

gl_receivedUSARTData[gl_receivedUSARTPointer++] = RCREG;

if (gl_receivedUSARTPointer>=USARTBUFFERSIZE)gl_receivedUSARTPointer=0;

}

}

#pragma code

Can transmission is performed either by the user_putc() function when an ASCII message is to be sent to the master, or by a specific CAN routine to send compressed data.

/* ==============================================================================

* Function: CANsendDelay

* Returns: void = no return.

* Description: Small busy‑wait delay between CAN frames.

* ============================================================================== */

void CANsendDelay(void) {

short	delay;

for(delay=0;delay<WAITDELAYTRAMECAN;delay++);

}

/* ==============================================================================

* Function: sendPrintToCAN

* Returns: void = no return.

* Description: Packages the output buffer into CAN 'PRINT' frames with header/footer

* and sends.

* ============================================================================== */

void sendPrintToCAN(void){

long 	id;			// Id of sender

BYTE 	dataOut[8];	// DATA to CAN

char 	dataCounter;

char	dataOutCounter;

char	trameComplete;

BYTE dataLen; 				// Number of bytes transmitted in the gl_message

ECAN_TX_MSG_FLAGS flags; 	// Flags

dataLen=8;

flags=ECAN_TX_STD_FRAME;

id=gl_boardNumber;

// Watchdog off

WDTCONbits.SWDTEN = 0;

// header trame

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) dataOut[dataOutCounter]=TRAMEPRINTHEADER;

while(!ECANSendMessage(id,dataOut,dataLen,flags));

CANsendDelay();

// trame

trameComplete=FALSE;

dataCounter=0;

while(dataCounter<MAXMESSAGESIZE) {

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) {

if (dataCounter<gl_outputCANbufferCounter) {

dataOut[dataOutCounter]=gl_outputCANbuffer[dataCounter++];

if ((char)dataOut[dataOutCounter]==(char)ENDOFPRINTFTRAME) trameComplete=TRUE;

}

else {

dataOut[dataOutCounter]=ENDOFPRINTFTRAME;

if (dataCounter<MAXMESSAGESIZE)dataCounter++;

trameComplete=TRUE;

}

}

while(!ECANSendMessage(id,dataOut,dataLen,flags));

CANsendDelay();

if ((char)trameComplete==(char)TRUE) break;

}

gl_outputCANbufferCounter=0;

// footer trame

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) dataOut[dataOutCounter]=TRAMEPRINTFOOTER;

while(!ECANSendMessage(id,dataOut,dataLen,flags));

CANsendDelay();

// Watchdog on

WDTCONbits.SWDTEN = 1;

}

/* ==============================================================================

* Function: _user_putc

* Returns: int = the character written.

* Description: Writes a character to UART on master or queues to CAN on slave;

* triggers CAN send on buffer/full or terminator.

* ============================================================================== */

int _user_putc (char c) {

// On master board send to UART via standart putc()

if ((char)gl_master==(char)TRUE){

if (gl_inputCounter==0)sendUSART(c);

}

// Send on CAN bus

else {

if (gl_outputCANbufferCounter<MAXMESSAGESIZE)gl_outputCANbuffer[gl_outputCANbufferCounter++]=c;

if ((gl_outputCANbufferCounter==MAXMESSAGESIZE || c==ENDOFPRINTFTRAME)) {

sendPrintToCAN();

}

}

return(c);

}

/* ==============================================================================

* Function: sendRequestToCAN

* Returns: void = no return.

* Description: Compresses gl_request and transmits it as a CAN 'REQUEST' frame

* sequence; restores request via RLE decode.

* ============================================================================== */

void sendRequestToCAN(void) {

long 	id;			// Id of sender

BYTE 	dataOut[8];	// DATA to CAN

char 	dataCounter;

char	dataOutCounter;

BYTE dataLen; 				// Number of bytes transmitted in the gl_message

ECAN_TX_MSG_FLAGS flags; 	// Flags

char trameSize;

// Convert request to dataOut using gl_dataStructure

trameSize=compressData();

dataLen=8;

flags=ECAN_TX_STD_FRAME;

dataCounter=0;

id=gl_boardNumber;

// header trame

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) {

dataOut[dataOutCounter]=TRAMEREQUESTHEADER;

}

while(!ECANSendMessage(id,dataOut,dataLen,flags));

CANsendDelay();

// trame

while(dataCounter<trameSize) {

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) {

if (dataCounter<trameSize) {

dataOut[dataOutCounter]=gl_request[dataCounter++];

if(dataCounter>=MAXTRAMESIZE) return; // Something wrong happened

}

else {

dataOut[dataOutCounter]=0;

}

}

while(!ECANSendMessage(id,dataOut,dataLen,flags));

CANsendDelay();

}

// footer trame

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) {

dataOut[dataOutCounter]=TRAMEREQUESTFOOTER;

}

while(!ECANSendMessage(id,dataOut,dataLen,flags));

uncompressData(); // get back to initial data for other action in manageRequest()

}

/* ==============================================================================

* Function: CAN_SendSync

* Returns: void = no return.

* Description: Broadcasts a CAN SYNC frame and re‑phases timers and counters

* across boards.

* ============================================================================== */

void CAN_SendSync(void){

BYTE dataLen; 				// Number of bytes transmitted in the gl_message

ECAN_TX_MSG_FLAGS flags; 	// Flags

BYTE 	dataOut[8];	// DATA to CAN

char	dataOutCounter;

dataLen=8;

flags=ECAN_TX_STD_FRAME;

// sync trame

for(dataOutCounter=0;dataOutCounter<8;dataOutCounter++) {

dataOut[dataOutCounter]=TRAMESYNCTRACE;

}

while (!ECANSendMessage(SYNC_ID, dataOut, dataLen, flags));

gl_mutexLowIsr = 1;

INTCONbits.TMR0IE       = 0;   // Disable Timer0 Interrupt

gl_speedCounter			= 1;   // Same value on each board

gl_syncRequested 		= 1;   // For printing SYNC message on LED

INTCONbits.TMR0IF       = 0;   // T0 int flag bit cleared before starting

TMR0H					= 0;   // re-phase Timer0

TMR0L 					= 0;   // re-phase Timer0

INTCONbits.TMR0IE       = 1;   // Enable Timer0 Interrupt

T0CONbits.TMR0ON        = 1;   // timer0 START

gl_mutexLowIsr = 0;

CANsendDelay();

}

/* ==============================================================================

* Function: getInputRequestFromCAN

* Returns: char = TRUE when a request was assembled, FALSE otherwise.

* Description: Parses incoming CAN buffers, extracting REQUEST or PRINT trames;

* fills gl_request or prints content.

* ============================================================================== */

char getInputRequestFromCAN(void) {

char	requestHeaderTrameDetected;

char	printHeaderTrameDetected;

char	requestFooterTrameDetected;

char	printFooterTrameDetected;

char	requestTrameEnd;

char	printTrameEnd;

char	dataInCounter;

char	dataStructureCounter;

char 	bufferNumber;

char	data;

mySprintf((char *)gl_message,"");

for(bufferNumber=0;bufferNumber<MAXINPUTCANBUFFER;bufferNumber++) {

requestHeaderTrameDetected=0;

printHeaderTrameDetected=0;

requestFooterTrameDetected=0;

printFooterTrameDetected=0;

while (gl_inputCANReadBufferPointer[bufferNumber]!=gl_inputCANWriteBufferPointer[bufferNumber]) {

data=gl_inputCANbuffer[bufferNumber][gl_inputCANReadBufferPointer[bufferNumber]];

if ((char)data==(char)TRAMEREQUESTHEADER) requestHeaderTrameDetected++;	else requestHeaderTrameDetected=0;

if ((char)data==(char)TRAMEREQUESTFOOTER) requestFooterTrameDetected++;	else requestFooterTrameDetected=0;

if ((char)data==(char)TRAMEPRINTHEADER) printHeaderTrameDetected++;		else printHeaderTrameDetected=0;

if ((char)data==(char)TRAMEPRINTFOOTER) printFooterTrameDetected++;		else printFooterTrameDetected=0;

// REQUEST HEADER

if (requestHeaderTrameDetected==8) {

gl_inputCANmode[bufferNumber]=CAN_REQUEST;

requestHeaderTrameDetected=0;

gl_requestInputCANtrameStart[bufferNumber]=gl_inputCANReadBufferPointer[bufferNumber]+1;

if ((int)gl_requestInputCANtrameStart[bufferNumber]>=MAXTRAMESIZE)gl_requestInputCANtrameStart[bufferNumber]=0;

}

else if (printHeaderTrameDetected==8) {

gl_inputCANmode[bufferNumber]=CAN_PRINT;

printHeaderTrameDetected=0;

gl_printInputCANtrameStart[bufferNumber]=gl_inputCANReadBufferPointer[bufferNumber]+1;

if ((char)gl_printInputCANtrameStart[bufferNumber]>=(char)MAXTRAMESIZE)gl_printInputCANtrameStart[bufferNumber]=0;

}

else if (requestFooterTrameDetected==8 && gl_inputCANmode[bufferNumber]==CAN_REQUEST) {

gl_inputCANmode[bufferNumber]=CAN_FREE;

requestTrameEnd=gl_inputCANReadBufferPointer[bufferNumber]-7;

if (requestTrameEnd<0)requestTrameEnd+=MAXTRAMESIZE;

dataInCounter=gl_requestInputCANtrameStart[bufferNumber];

dataStructureCounter=0;

while(dataInCounter!=requestTrameEnd) {

gl_request[dataStructureCounter++]=gl_inputCANbuffer[bufferNumber][dataInCounter++];

if (dataInCounter>=MAXTRAMESIZE)dataInCounter=0;

if (dataStructureCounter>=REQUESTSIZE){

return(FALSE);

}

}

gl_inputCANReadBufferPointer[bufferNumber]++;

if (gl_inputCANReadBufferPointer[bufferNumber]>=MAXTRAMESIZE)gl_inputCANReadBufferPointer[bufferNumber]=0;

return(uncompressData()); // Mean request available to proceed

}

else if (printFooterTrameDetected==8 && gl_inputCANmode[bufferNumber]==CAN_PRINT) {

gl_inputCANmode[bufferNumber]=CAN_FREE;

printTrameEnd=gl_inputCANReadBufferPointer[bufferNumber]-7;

if (printTrameEnd<0)printTrameEnd+=MAXTRAMESIZE;

dataInCounter=gl_printInputCANtrameStart[bufferNumber];

// Watchdog off

WDTCONbits.SWDTEN = 0;

while(dataInCounter!=printTrameEnd) {

if ((char)gl_master==(char)TRUE && (char)gl_inputCANbuffer[bufferNumber][dataInCounter]!=(char)0) _user_putc(gl_inputCANbuffer[bufferNumber][dataInCounter]);

dataInCounter++;

if (dataInCounter>=MAXTRAMESIZE)dataInCounter=0;

}

// Watchdog on

WDTCONbits.SWDTEN = 1;

gl_inputCANReadBufferPointer[bufferNumber]++;

if (gl_inputCANReadBufferPointer[bufferNumber]>=MAXTRAMESIZE)gl_inputCANReadBufferPointer[bufferNumber]=0;

return(FALSE); // Mean no more data to print

}

// read new data

gl_inputCANReadBufferPointer[bufferNumber]++;

if (gl_inputCANReadBufferPointer[bufferNumber]>=MAXTRAMESIZE)gl_inputCANReadBufferPointer[bufferNumber]=0;

}

}

return(FALSE); // Nothing to do

}

## PWM management in analog mode, DCC management in NMRA digital mode and TM1637 device management

PWM or DCC signal management is performed under interrupt, including TM1637 7 segment display management

/* ==============================================================================

* Function: set7segmentPort

* Returns: void = no return.

* Description: Sets the TM1637 lines (CLK,DIO) on a shared port mapping.

* ============================================================================== */

void set7segmentPort(char CLK, char DIO) {

char myPortE; // used to better synchronised output updates

char delay;

myPortE= (CLK<<1) + (DIO<<2);

LATE=myPortE;

}

/* ==============================================================================

* Function: twoWire_init

* Returns: void = no return.

* Description: Initializes pseudo two‑wire interface for TM1637.

* ============================================================================== */

void twoWire_init(void) {

set7segmentPort(0,0);

}

/* ==============================================================================

* Function: twoWire_start

* Returns: void = no return.

* Description: Generates TM1637 start sequence.

* ============================================================================== */

void twoWire_start(void){

set7segmentPort(1,1);

set7segmentPort(1,0);

}

/* ==============================================================================

* Function: twoWire_stop

* Returns: void = no return.

* Description: Generates TM1637 stop sequence.

* ============================================================================== */

void twoWire_stop(void){

set7segmentPort(0,0);

set7segmentPort(1,0);

set7segmentPort(1,1);

}

/* ==============================================================================

* Function: twoWire_ack

* Returns: void = no return.

* Description: Generates a TM1637 dummy ACK pulse.

* ============================================================================== */

void twoWire_ack(void){

set7segmentPort(0,0);

set7segmentPort(1,0);

}

/* ==============================================================================

* Function: twoWire_write

* Returns: char = unspecified / not used.

* Description: Clocks out 8 bits LSB‑first on TM1637 (bit‑bang).

* ============================================================================== */

char twoWire_write(char data){

char tx;

char DIO;

for(tx = 0 ; tx < 8 ; tx++) {

DIO = ((data >> tx) & 0x01) ? 1 : 0 ; //LSB first (Real 12c sends MSB first)

set7segmentPort(0,DIO);

set7segmentPort(1,DIO);

set7segmentPort(0,DIO);

}

}

/* ==============================================================================

* Function: TM1637_init

* Returns: void = no return.

* Description: Initializes the TM1637 display driver.

* ============================================================================== */

void TM1637_init(void){

twoWire_init();

}

/* ==============================================================================

* Function: TM1637_write

* Returns: void = no return.

* Description: Converts two 3‑digit numbers to 6 digits and writes segment bytes.

* ============================================================================== */

void TM1637_write(short number1,short number2){

char str1Num[4];

char str2Num[4];

char strNum[8];

char size;

mySprintf(str1Num,"%3d",number1);

mySprintf(str2Num,"%3d",number2); // 3 characters

mySprintf(strNum,"%s%s",str2Num,str1Num);

for(size=5;size>=0;size--) {

if ((char)strNum[size]==(char)' ')twoWire_write(digits[11]);

else if ((char)strNum[size]==(char)'-')twoWire_write(digits[10]);

else {

char i = strNum[size] - '0';  //Get index 0 - 9

twoWire_write(digits[i]);

}

twoWire_ack();

}

}

/* ==============================================================================

* Function: TM1637_display

* Returns: void = no return.

* Description: Configures TM1637 and displays two numbers with given brightness.

* ============================================================================== */

void TM1637_display(short number1,short number2){

TM1637_setBrightness(4);

twoWire_start();

twoWire_write(0x40);

twoWire_ack();

twoWire_stop();

twoWire_start();

twoWire_write(0xC0);

twoWire_ack();

TM1637_write(number1,number2);

twoWire_stop();

}

/* ==============================================================================

* Function: charToSegments

* Returns: char = segment bitmap.

* Description: Maps ASCII char to 7‑segment encoding for common‑anode TM1637.

* ============================================================================== */

char charToSegments(char c) {

if((char)c >= (char)'0' && (char)c <= (char)'9')        return digits[c - '0'];

else if((char)c == (char)'-')               return digits[10];

else if((char)c == (char)' ')               return digits[11];

else if((char)c >= (char)'A' && (char)c <= (char)'Z')   return digits[12 + (c - 'A')];

else if((char)c >= (char)'a' && (char)c <= (char)'z')   return digits[12 + (c - 'a')];

else                            return 0x00;

}

/* ==============================================================================

* Function: TM1637_writeStringWindow

* Returns: void = no return.

* Description: Writes exactly 6 characters to TM1637 using physical position mapping.

* ============================================================================== */

void TM1637_writeStringWindow(const char *s, char start, char len) {

// physical order of the positions on screen (0 = left, 5 = right)

// adjust this array if the order is different

const char mapping[6] = {2,1,0,5,4,3};

char i;

for (i = 0; i < 6; i++) {

char idx = start + mapping[i];

char ch;

if (idx < len) ch = s[idx];

else ch = ' ';

twoWire_write(charToSegments(ch));

twoWire_ack();

}

}

/* ==============================================================================

* Function: TM1637_displayString

* Returns: void = no return.

* Description: Displays a string on 6 digits; scrolls if longer than 6.

* ============================================================================== */

void TM1637_displayString(char *string) {

char len;

char start;

// init

len = strlen(string);

TM1637_setBrightness(4);

if (len <= 6) {

twoWire_start();

twoWire_write(0x40);

twoWire_ack();

twoWire_stop();

twoWire_start();

twoWire_write(0xC0);

twoWire_ack();

TM1637_writeStringWindow(string, 0, len);

twoWire_stop();

}

else {

for (start = 0; start <= (len - 6); start++) {

twoWire_start();

twoWire_write(0x40);

twoWire_ack();

twoWire_stop();

twoWire_start();

twoWire_write(0xC0);

twoWire_ack();

TM1637_writeStringWindow(string, start, len);

twoWire_stop();

delayMainLoop(1);

}

}

}

/* ==============================================================================

* Function: TM1637_setBrightness

* Returns: void = no return.

* Description: Sets TM1637 brightness (0..8). 0 turns display off.

* ============================================================================== */

void TM1637_setBrightness(char level){

twoWire_start();

twoWire_write(0x87 + level);

twoWire_ack();

twoWire_stop();

}

/* ==============================================================================

* Function: setPort

* Returns: void = no return.

* Description: Updates microcontroller output latches (A..D) from current state

* variables; handles stopAll.

* ============================================================================== */

void setPort(void){

char myPortA; // used to better synchronised output updates

char myPortB; // used to better synchronised output updates

char myPortC; // used to better synchronised output updates

char myPortD; // used to better synchronised output updates

if ((char)gl_stopAll==(char)TRUE) {

gl_S1T0char=0; gl_S2T0char=0;

gl_S1T1char=0; gl_S2T1char=0;

gl_S1T2char=0; gl_S2T2char=0;

gl_S1T3char=0; gl_S2T3char=0;

LATA=0xFF;

LATB=0xFF;

LATC=0xFF;

LATD=0xFF;

}

else {

myPortA=(gl_S1T0char<<4) + (gl_S1T1char<<6) + (gl_S2T0char<<7);

myPortB=(gl_OUTchar[2]&1) + ((gl_OUTchar[3]&1)<<1) + ((gl_OUTchar[4]&1)<<4) + ((gl_OUTchar[5]&1)<<5);

myPortC=(gl_S2T1char) + (gl_S1T2char<<1) + (gl_S2T2char<<2) + (gl_S1T3char<<3) +((char)TRISCbits.RC4==(char)0 ? gl_GPIOchar[3] <<4 : 0);

myPortD=(gl_S2T3char) + ((char)TRISDbits.RD1==(char)0 ? gl_GPIOchar[0] <<1 :0) + ((char)TRISDbits.RD2==(char)0 ? gl_GPIOchar[1] <<2:0) +((char)TRISDbits.RD3==(char)0 ? gl_GPIOchar[2] <<3:0) + ((gl_OUTchar[0]&1)<<6) +  ((gl_OUTchar[1]&1)<<7);

LATA=myPortA;

LATC=myPortC;

LATD=myPortD;

LATB=myPortB;

}

}

/********************************************************************************

* low_interrupt

********************************************************************************/

#pragma code low_vector=0x18

/* ==============================================================================

* Function: low_interrupt

* Returns: void = vector stub.

* Description: Low‑priority interrupt vector stub that jumps to low_isr.

* ============================================================================== */

void low_interrupt (void){

_asm goto low_isr _endasm

}

#pragma code

#pragma interruptlow low_isr

/* ==============================================================================

* Function: low_isr_task

* Returns: void = no return.

* Description: Low‑priority background task called from ISR: manages flashing,

* reads knobs/ADC and computes averages.

* ============================================================================== */

void low_isr_task(void) {

char 	bitStateCounter;

char 	delay;

char 	selectBitDelay;

char 	bitNumber;

char 	bitValue;

short 	ADC;

int 	knobValue1;

if (gl_goFlashingCounter==1)gl_flashingCounter++;

if (((gl_flashingCounter & 0xFFF) == 0) || ((gl_flashingCounter & 0xFFF) == 0x7FF))gl_goFlashingCounter=0;

// KNOBS MANAGEMENT

gl_getKnobValue++;

if (gl_getKnobValue==0) {

gl_numberKnobData++;

// KNOB VALUE

ADCON0=INKNOB0;

ADCON0bits.GO = 1;                            // ADCON0.GODONE = 1

while((char)ADCON0bits.GO == (char)1);        // wait till GODONE bit is zero

ADC = ADRESH;    //Read converted result

ADC = (ADC<<8) + ADRESL;

gl_adcKnobValue0+=ADC;

ADCON0=INKNOB1;

ADCON0bits.GO = 1;                            // ADCON0.GODONE = 1

while((char)ADCON0bits.GO == (char)1);        // wait till GODONE bit is zero

ADC = ADRESH;    //Read converted result

ADC = (ADC<<8) + ADRESL;

gl_adcKnobValue1+=ADC;

if(gl_numberKnobData>=20) {

gl_numberKnobData=0;

gl_adcKnobValue0=gl_adcKnobValue0/10;

gl_adcKnobValue1=gl_adcKnobValue1/10;

if (gl_calibKnob==1) {

if (gl_minAdcKnobValue0>gl_adcKnobValue0)gl_minAdcKnobValue0=gl_adcKnobValue0;

if (gl_minAdcKnobValue1>gl_adcKnobValue1)gl_minAdcKnobValue1=gl_adcKnobValue1;

if (gl_maxAdcKnobValue0<gl_adcKnobValue0)gl_maxAdcKnobValue0=gl_adcKnobValue0;

if (gl_maxAdcKnobValue1<gl_adcKnobValue1)gl_maxAdcKnobValue1=gl_adcKnobValue1;

gl_deltaKnob0=gl_maxAdcKnobValue0-gl_minAdcKnobValue0;

gl_deltaKnob1=gl_maxAdcKnobValue1-gl_minAdcKnobValue1;

}

// Speed

gl_knobValue0=(31*((long)(gl_adcKnobValue0-gl_minAdcKnobValue0))/(long)(gl_deltaKnob0))-15;

if (gl_knobValue0>15)gl_knobValue0=15;

else if (gl_knobValue0==-1 || gl_knobValue0==1)gl_knobValue0=0; // For stability around 0

// Inertia

knobValue1=(101*(long)(gl_adcKnobValue1-gl_minAdcKnobValue1))/(long)(gl_deltaKnob1);

if (knobValue1>100)knobValue1=100;

if(gl_knobValue1==0 && knobValue1==100)knobValue1=0; // Should happen if potentiometer lost contact

gl_knobValue1=knobValue1;

gl_adcKnobValue0=0;

gl_adcKnobValue1=0;

}

}

// GPIO IN Detection

if((char)TRISDbits.RD1==(char)1 && (char)PORTDbits.RD1!=(char)gl_GPIOchar[0]){

gl_GPIOstabilized[0]++;

if (gl_GPIOstabilized[0]>(unsigned int)GPIOTHRESHOLD) {

gl_GPIOchar[0]=PORTDbits.RD1;

if (gl_GPIOchar[0]==1)gl_GPIOcounter[0]++;

gl_GPIONotification[0]=TRUE;

gl_GPIOstabilized[0]=0;

}

}

else gl_GPIOstabilized[0]=0;

if((char)TRISDbits.RD2==(char)1 && (char)PORTDbits.RD2!=(char)gl_GPIOchar[1]){

gl_GPIOstabilized[1]++;

if (gl_GPIOstabilized[1]>(unsigned int)GPIOTHRESHOLD) {

gl_GPIOchar[1]=PORTDbits.RD2;

if (gl_GPIOchar[1]==1)gl_GPIOcounter[1]++;

gl_GPIONotification[1]=TRUE;

gl_GPIOstabilized[1]=0;

}

}

else gl_GPIOstabilized[1]=0;

if((char)TRISDbits.RD3==(char)1 && (char)PORTDbits.RD3!=(char)gl_GPIOchar[2]){

gl_GPIOstabilized[2]++;

if (gl_GPIOstabilized[2]>(unsigned int)GPIOTHRESHOLD) {

gl_GPIOchar[2]=PORTDbits.RD3;

if (gl_GPIOchar[2]==1)gl_GPIOcounter[2]++;

gl_GPIONotification[2]=TRUE;

gl_GPIOstabilized[2]=0;

}

}

else gl_GPIOstabilized[2]=0;

if((char)TRISCbits.RC4==(char)1 && (char)PORTCbits.RC4!=(char)gl_GPIOchar[3]){

gl_GPIOstabilized[3]++;

if (gl_GPIOstabilized[3]>(unsigned int)GPIOTHRESHOLD) {

gl_GPIOchar[3]=PORTCbits.RC4;

if (gl_GPIOchar[3]==1)gl_GPIOcounter[3]++;

gl_GPIONotification[3]=TRUE;

gl_GPIOstabilized[3]=0;

}

}

else gl_GPIOstabilized[3]=0;

if ((char)gl_stopAll==(char)FALSE) {

// TIMER

gl_timer--;

if ((unsigned short)gl_timer==(unsigned short)0) {

gl_timer=INITTIMERVALUE;

if (gl_TIMERValue[gl_timerNumber]>0){

gl_TIMERValue[gl_timerNumber]--;

if (gl_TIMERValue[gl_timerNumber]==0)gl_TIMERNotification[gl_timerNumber]=TRUE;

}

gl_timerNumber++;

if(gl_timerNumber>MAXTIMER)gl_timerNumber=0;

}

gl_trackNumber=gl_trackNumber+1;

if (gl_trackNumber>3)gl_trackNumber=0;

//////////// MODE ANALOG ////////////////

if(gl_boardMode==ANAValue) {

gl_speedCounter++;

if (gl_speedCounter>MAX_INERTIA_COUNTER) {

gl_speedCounter=1;

}

if (gl_curSpeed[gl_trackNumber]!=gl_setPoint[gl_trackNumber]) {

gl_setStepCounter[gl_trackNumber]--;

if (gl_setStepCounter[gl_trackNumber]<=0) {

gl_setStepCounter[gl_trackNumber]=gl_setStep[gl_trackNumber];

if (gl_curSpeed[gl_trackNumber]>gl_setPoint[gl_trackNumber]){

gl_curSpeed[gl_trackNumber]-=MAX_STEP;

if(gl_curSpeed[gl_trackNumber]<gl_setPoint[gl_trackNumber])gl_curSpeed[gl_trackNumber]=gl_setPoint[gl_trackNumber];

}

else {

gl_curSpeed[gl_trackNumber]+=MAX_STEP;

if(gl_curSpeed[gl_trackNumber]>gl_setPoint[gl_trackNumber])gl_curSpeed[gl_trackNumber]=gl_setPoint[gl_trackNumber];

}

}

if (gl_curSpeed[gl_trackNumber]>0) {

gl_speed[gl_trackNumber]=gl_curSpeed[gl_trackNumber]/(MAXINTERNALSPEED);

gl_direction[gl_trackNumber]=TRACK_BACKWARD;

}

else if (gl_curSpeed[gl_trackNumber]<0) {

gl_speed[gl_trackNumber]=-gl_curSpeed[gl_trackNumber]/(MAXINTERNALSPEED);

gl_direction[gl_trackNumber]=TRACK_FORWARD;

}

else {

gl_direction[gl_trackNumber]=TRACK_STOP;

gl_speed[gl_trackNumber]=0;

}

}

if (gl_speed[gl_trackNumber]>=gl_speedCounter) {

if (gl_direction[gl_trackNumber]==TRACK_FORWARD) {

switch (gl_trackNumber) {

case 0:gl_S1T0char=1; gl_S2T0char=0;break;

case 1:gl_S1T1char=1; gl_S2T1char=0;break;

case 2:gl_S1T2char=1; gl_S2T2char=0;break;

case 3:gl_S1T3char=1; gl_S2T3char=0;break;

}

}

if (gl_direction[gl_trackNumber]==TRACK_BACKWARD) {

switch (gl_trackNumber) {

case 0:gl_S1T0char=0; gl_S2T0char=1;break;

case 1:gl_S1T1char=0; gl_S2T1char=1;break;

case 2:gl_S1T2char=0; gl_S2T2char=1;break;

case 3:gl_S1T3char=0; gl_S2T3char=1;break;

}

}

if (gl_direction[gl_trackNumber]==TRACK_STOP) {

switch (gl_trackNumber) {

case 0:gl_S1T0char=0; gl_S2T0char=0;break;

case 1:gl_S1T1char=0; gl_S2T1char=0;break;

case 2:gl_S1T2char=0; gl_S2T2char=0;break;

case 3:gl_S1T3char=0; gl_S2T3char=0;break;

}

}

}

else {

switch (gl_trackNumber) {

case 0:gl_S1T0char=0; gl_S2T0char=0;break;

case 1:gl_S1T1char=0; gl_S2T1char=0;break;

case 2:gl_S1T2char=0; gl_S2T2char=0;break;

case 3:gl_S1T3char=0; gl_S2T3char=0;break;

}

}

setPort();

}

else if(gl_boardMode==DCCValue && gl_dcc_ready==0) {

for (bitStateCounter=0;bitStateCounter<FRAME_SIZE;bitStateCounter++) {

if (gl_dcc[bitStateCounter]==0) selectBitDelay=DCC_0;

else selectBitDelay=DCC_1;

gl_S1T0char=0;gl_S1T1char=0;

gl_S1T2char=0;gl_S1T3char=0;

gl_S2T0char=1;gl_S2T1char=1;

gl_S2T2char=1;gl_S2T3char=1;

setPort();

for (delay=0;delay<selectBitDelay;delay++);

gl_S2T0char=0;gl_S2T1char=0;

gl_S2T2char=0;gl_S2T3char=0;

gl_S1T0char=1;gl_S1T1char=1;

gl_S1T2char=1;gl_S1T3char=1;

setPort();

for (delay=0;delay<selectBitDelay;delay++);

}

gl_S1T0char=0; gl_S2T0char=1;

gl_S1T1char=0; gl_S2T1char=1;

gl_S1T2char=0; gl_S2T2char=1;

gl_S1T3char=0; gl_S2T3char=1;

setPort();

for (delay=0;delay<selectBitDelay;delay++);

for (bitStateCounter=0;bitStateCounter<FRAME_SIZE;bitStateCounter++)gl_dcc[bitStateCounter]=1;

}

gl_dcc_ready--;

if (gl_dcc_ready<0) gl_dcc_ready=INITWAITDCCCOUNTER;

// TRACK DETECTION

switch(gl_trackNumber) {

case 0 : ADCON0=CURT0;break;

case 1 : ADCON0=CURT1;break;

case 2 : ADCON0=CURT2;break;

case 3 : ADCON0=CURT3;break;

}

// NEED TO GET LOW VOLTAGE VALUE WHEN TRACK IS OFF FOR CALIBRATION AT POWER ON

if ((char)gl_trackCalibration==(char)TRUE) {

ADCON0bits.GO = 1;                            // ADCON0.GODONE = 1

while((char)ADCON0bits.GO ==(char) 1);                    // wait till GODONE bit is zero

ADC = 0;

ADC = ADRESH;    //Read converted result

ADC = (ADC<<8) + ADRESL;

gl_average[gl_trackNumber]=(SAMPLEFORCALIBRATION*gl_average[gl_trackNumber]+ADC)/(SAMPLEFORCALIBRATION+1);

if (gl_noVehicule[gl_trackNumber]>gl_average[gl_trackNumber])gl_noVehicule[gl_trackNumber]=gl_average[gl_trackNumber];

}

if(((char)gl_speed[gl_trackNumber]==(char)gl_speedCounter && (char)gl_boardMode==(char)ANAValue) || ((char)gl_dcc_ready==(char)INITWAITDCCCOUNTER && (char)gl_boardMode==(char)DCCValue)) { // ONLY WHEN POWER ON

ADCON0bits.GO = 1;                            // ADCON0.GODONE = 1

while((char)ADCON0bits.GO == (char)1);                    // wait till GODONE bit is zero

ADC = 0;

ADC = ADRESH;    //Read converted result

ADC = (ADC<<8) + ADRESL;

if (gl_average[gl_trackNumber]<ADC) gl_average[gl_trackNumber]=ADC; // TRAP THE EVENT

else gl_average[gl_trackNumber]=(SAMPLEFORAVERAGE*gl_average[gl_trackNumber]+ADC)/(SAMPLEFORAVERAGE+1);

if (((int)(10*gl_average[gl_trackNumber])>(int)((10+HYSTERERISHIGH)*gl_noVehicule[gl_trackNumber])) && ((char)gl_OUTSTATchar[gl_trackNumber]==(char)0)  && ((char)gl_trackNotification[gl_trackNumber]==(char)FALSE)) {

gl_OUTSTATchar[gl_trackNumber]=1;

gl_trackNotification[gl_trackNumber]=TRUE;

}

else if (((int)(10*gl_average[gl_trackNumber])<(int)((10+HYSTERERISLOW)*gl_noVehicule[gl_trackNumber])) && ((char)gl_OUTSTATchar[gl_trackNumber]==(char)1) && ((char)gl_trackNotification[gl_trackNumber]==(char)FALSE)) {

gl_OUTSTATchar[gl_trackNumber]=0;

gl_trackNotification[gl_trackNumber]=TRUE;

}

}

}

else setPort();

}

/* ==============================================================================

* Function: low_isr

* Returns: void = no return.

* Description: Call low interrupt code

* ============================================================================== */

void low_isr(void){

if (!gl_mutexLowIsr) {

low_isr_task();

}

// INTERRUPT RESET

if((char)INTCONbits.TMR0IF==(char)1){

INTCONbits.TMR0IF = 0;

T0CONbits.PSA			= 0;   // Timer0 prescaler is assigned

T0CONbits.T0PS0			= 0;   // Prescale value

T0CONbits.T0PS1			= 0;   // Prescale value

T0CONbits.T0PS2			= 0;   // Prescale value

}

}

#pragma code

The DCC frame in standard NMRA format is generated by the setDcc() function and sent by low_isr(void) shown above . Please note that the timings have been adjusted by the following define values:

// DELAY FOR DCC SIGNAL

#define DCC_0 48

#define DCC_1 18

Any modification to the code, such as the use of int instead of char for example, may modify the timing and cause the DCC signal to become invalid. In this case, it is necessary to check the correct timing values of the DCC signal with an oscilloscope (58us for a 1 and 100us for a 0).

/* ==============================================================================

* Function: setDcc

* Returns: void = no return.

* Description: Builds a full DCC packet from address and command into gl_dcc buffer.

* ============================================================================== */

void setDcc(char address, char command) {

char i;

char bitNumber;

char control;

control=address ^ command; // Control

bitNumber=0;

// PREAMBULE

for(i=0;i<PREAMBLE_SIZE;i++) {

gl_dcc[bitNumber++]=1;

}

// 0

gl_dcc[bitNumber++]=0;

// ADDRESS

for(i=0;i<8;i++) {

gl_dcc[bitNumber++]=address >> (7-i) & 1;

}

// 0

gl_dcc[bitNumber++]=0;

// COMMAND

for(i=0;i<8;i++) {

gl_dcc[bitNumber++]=command >> (7-i) & 1;

}

// 0

gl_dcc[bitNumber++]=0;

// CONTROL

for(i=0;i<8;i++) {

gl_dcc[bitNumber++]=control >> (7-i) & 1;

}

// 1

gl_dcc[bitNumber++]=1;

gl_dcc[bitNumber++]=1; // Only one is enough, but in case of....

}

## Flash storage of programming information

EEPROM data storage is managed by a set of flash read/write functions. In order to get more space, automations are compressed inside EEPROM.

/* ==============================================================================

* Function: ReadEEPROM

* Returns: char = SUCCESS on read, ERROR if address invalid.

* Description: Reads one byte from data EEPROM at the given address into *data.

* Checks bounds and selects data EEPROM space.

* ============================================================================== */

char ReadEEPROM(int adr, char *data){

if(adr > 0x3FF){

return(ERROR);

}

else{

EEADR = adr&0xFF;

EEADRH = (adr>>8) & 0x3;

EECON1bits.EEPGD = 0;	// Point to data memory

EECON1bits.CFGS = 0;	// Access EEPROM

EECON1bits.RD = 1;		// Read data

*data = EEDATA;			// Load data

return(SUCCESS);

}

}

/* ==============================================================================

* Function: ResetEEPROM

* Returns: void = no return.

* Description: Initializes EEPROM to default values (magic number, ANA mode,

* clears automation, sets GPIO directions) and syncs globals.

* ============================================================================== */

void ResetEEPROM(void){

char 	value;

short adr;

char  checkMagicNumberCounter;

// Init EEPROM after flashing the board

adr=(short)MAGICNUMBER_ADDRESS;

for (checkMagicNumberCounter=0;checkMagicNumberCounter<MAGICNUMBERSIZE;checkMagicNumberCounter++) {

WriteEEPROM(adr++,checkMagicNumberCounter);

}

// Set ANA mode

adr=(short)MODE_ADDRESS;

value=ANAValue;

WriteEEPROM(adr,value);

// No automation

adr=(short)NEXTTAUTOMATION_ADDRESS;

value=0;

WriteEEPROM(adr,value);

gl_mutexLowIsr=1;gl_boardMode = ANAValue;gl_nexAvailableAutomation=0;gl_mutexLowIsr=0;

// GPIO IN

for(adr=(short)GPIO0DIR_ADDRESS;adr<=(short)GPIO0DIR_ADDRESS+3;adr++)WriteEEPROM(adr,1);

}

/* ==============================================================================

* Function: ReadEEPROMConfig

* Returns: void = no return.

* Description: Loads configuration from EEPROM: validates magic, board mode,

* GPIO directions, knob calibration, and automation cache.

* ============================================================================== */

void ReadEEPROMConfig(void) {

char 	value;

char 	dummy;

short adr;

char  automationCounter;

char  automationDataCounter;

char  checkMagicNumberCounter;

// Read MAGIC NUMBER

adr=(char)MAGICNUMBER_ADDRESS;

for (checkMagicNumberCounter=0;checkMagicNumberCounter<MAGICNUMBERSIZE;checkMagicNumberCounter++) {

ReadEEPROM(adr++,&value);

if (value!=checkMagicNumberCounter) {

ResetEEPROM();

return;

}

}

// Read in EEPROM MODE

adr=(short)MODE_ADDRESS;

ReadEEPROM(adr,&value);

gl_mutexLowIsr=1;gl_boardMode=value;gl_mutexLowIsr=0;

// Read in GPIO dir

adr=(short)GPIO0DIR_ADDRESS;

ReadEEPROM(adr++,&value);

TRISDbits.RD1=value;

ReadEEPROM(adr++,&value);

TRISDbits.RD2=value;

ReadEEPROM(adr++,&value);

TRISDbits.RD3=value;

ReadEEPROM(adr,&value);

TRISCbits.RC4=value;

// Knob min & max

adr=(short)KNOB_ADDRESS;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_minAdcKnobValue0=value;gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_minAdcKnobValue0=value+(gl_minAdcKnobValue0<<8);gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_maxAdcKnobValue0=value;gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_maxAdcKnobValue0=value+(gl_maxAdcKnobValue0<<8);gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_minAdcKnobValue1=value;gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_minAdcKnobValue1=value+(gl_minAdcKnobValue1<<8);gl_mutexLowIsr=0;

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_maxAdcKnobValue1=value;gl_mutexLowIsr=0;

ReadEEPROM(adr,&value);

gl_mutexLowIsr=1;gl_maxAdcKnobValue1=value+(gl_maxAdcKnobValue1<<8);gl_mutexLowIsr=0;

gl_mutexLowIsr=1;gl_deltaKnob0=gl_maxAdcKnobValue0-gl_minAdcKnobValue0;gl_mutexLowIsr=0;

gl_mutexLowIsr=1;gl_deltaKnob1=gl_maxAdcKnobValue1-gl_minAdcKnobValue1;gl_mutexLowIsr=0;

// Read in EEPROM last automation

dummy=getAutomationFromEEPROM();

}

/* ==============================================================================

* Function: WriteCompletedEEPROM

* Returns: char = SUCCESS when complete, ERROR otherwise.

* Description: Reports whether an EEPROM write cycle has completed and clears flags.

* ============================================================================== */

char WriteCompletedEEPROM(void) {

if(PIR2bits.EEIF){

PIR2bits.EEIF=0;		// Clear write complete flag

EECON1bits.WREN = 0;	// Disable write

return(SUCCESS);		// Write to EEPROM completed

}

else {

return(ERROR);			// Write to EEPROM not completed

}

}

/* ==============================================================================

* Function: WriteRdyEEPROM

* Returns: char = SUCCESS when ready, ERROR when busy.

* Description: Indicates if EEPROM is ready for a new write (no write in progress).

* ============================================================================== */

char WriteRdyEEPROM(void){

if(!EECON1bits.WR) {

return(SUCCESS);	// New Write Enabled

}

else {

return(ERROR);		// new Write Disabled

}

}

/* ==============================================================================

* Function: WriteEEPROM

* Returns: char = SUCCESS on write, ERROR if address invalid.

* Description: Writes one byte to data EEPROM at the given address with unlock

* sequence; skips write if data unchanged.

* ============================================================================== */

char WriteEEPROM(short adr, char data){

char checkValue;

if(adr > 0x3FF){

return(ERROR);

}

else{

// Wait eeprom ready to be written

while (WriteRdyEEPROM()==(char) ERROR);

// Check if this value already exists at that address

ReadEEPROM(adr,&checkValue);

if (checkValue==data)return(SUCCESS);

EEADR = adr&0xFF;			// Address of the data in EEPROM

EEADRH = (adr>>8) & 0x3;	// Address of the data in EEPROM

EEDATA = data;				// Data to write in EEPROM

EECON1bits.EEPGD = 0;		// Point to data memory

EECON1bits.CFGS = 0;		// Access EEPROM

EECON1bits.WREN = 1;		// Enable write

INTCONbits.GIE	= 0;		// Disable Interrupt

EECON2 = 0x55;

EECON2 = 0x0AA;

EECON1bits.WR = 1;			// Begin write

// Wait data written

while (WriteCompletedEEPROM()==(char) IN_PROGRESS);

while (WriteRdyEEPROM()==(char) ERROR);

INTCONbits.GIE		= 1;	// Enable Interrupt

return(SUCCESS);

}

}

/* ==============================================================================

* Function: getAutomationFromEEPROM

* Returns: char = percentage 0..100 (100 if none).

* Description: Reads automation entries from EEPROM into RAM and returns free

* space percentage.

* ============================================================================== */

char getAutomationFromEEPROM(void) {

char  automationCounter;

char  automationDataCounter;

char  value;

short adr;

long		   percentage;

// Get next automation address

adr=(short)NEXTTAUTOMATION_ADDRESS;

ReadEEPROM(adr,&value);

gl_mutexLowIsr=1;gl_nexAvailableAutomation=value;gl_mutexLowIsr=0;

if (gl_nexAvailableAutomation==0) return((double)100); // nothing to do

// Read and uncompress automation from EEPROM

adr=(short)AUTOMATION_ADDRESS;

for(automationCounter=0;automationCounter<gl_nexAvailableAutomation;automationCounter++) {

for(automationDataCounter=0;automationDataCounter<NEW_AUTOMATIONSIZE;automationDataCounter++) {

ReadEEPROM(adr++,&value);

gl_mutexLowIsr=1;gl_automation[automationCounter][automationDataCounter]=value;gl_mutexLowIsr=0;

}

}

percentage=100*(1024-(long)adr)/(1024-(long)AUTOMATION_ADDRESS);

return((char)percentage);

}

/* ==============================================================================

* Function: setAutomationToEEPROM

* Returns: char = TRUE on success, FALSE on overflow.

* Description: Writes automation entries from RAM to EEPROM and updates the

* 'next automation' pointer.

* ============================================================================== */

char setAutomationToEEPROM(void) {

char  automationCounter;

char  automationDataCounter;

short adr;

char  value;

adr=(short)AUTOMATION_ADDRESS;

for(automationCounter=0;automationCounter<gl_nexAvailableAutomation;automationCounter++) {

if (adr+NEW_AUTOMATIONSIZE>=1024) {

mySprintf((char *)gl_errorInfo,"limit is %d",(int)gl_nexAvailableAutomation);

gl_parserErrorCode=AUTOMATIONSIZELIMIT;

return(FALSE);

}

for(automationDataCounter=0;automationDataCounter<NEW_AUTOMATIONSIZE;automationDataCounter++) {

value=gl_automation[automationCounter][automationDataCounter];

WriteEEPROM(adr++,value);

}

}

// update in EEPROM next automation value

adr=(short)NEXTTAUTOMATION_ADDRESS;

value=gl_nexAvailableAutomation;

WriteEEPROM(adr,value);

return(TRUE);

}

/* ==============================================================================

* Function: uncompressData

* Returns: char = TRUE on success, FALSE on overflow.

* Description: Run‑length decodes gl_request buffer from (count,value) pairs

* into raw bytes.

* ============================================================================== */

char uncompressData(void) {

char dataCounter;

char tmpDataCounter;

char repeat;

// Codage is simply a list of (X,Y) where X is the number of Y. If X = 0 there is no more data

tmpDataCounter=0;

for(dataCounter=0;dataCounter<REQUESTSIZE;dataCounter++) gl_tmpBuffer[dataCounter]=0;

for(dataCounter=0;dataCounter<MAXTRAMESIZE-2;dataCounter+=2) {

if(gl_request[dataCounter]==0) break;

for(repeat=0;repeat<gl_request[dataCounter];repeat++) {

if (tmpDataCounter>=REQUESTSIZE) {

initRequest();

return(FALSE);

}

gl_tmpBuffer[tmpDataCounter++]=gl_request[dataCounter+1];

}

}

for(dataCounter=0;dataCounter<REQUESTSIZE;dataCounter++) gl_request[dataCounter]=gl_tmpBuffer[dataCounter];

return(TRUE);

}

/* ==============================================================================

* Function: compressData

* Returns: char = number of bytes produced (0 on failure).

* Description: Run‑length encodes gl_request into (count,value) pairs for

* CAN transmission.

* ============================================================================== */

char compressData(void) {

char tmpDataCounter;

char dataCounter;

char quantityValue;

char curValue;

// Init

dataCounter=0;

tmpDataCounter=0;

curValue=gl_request[0];

quantityValue=0;

while(dataCounter<REQUESTSIZE) {

while(gl_request[dataCounter]==curValue) {

quantityValue++;

dataCounter++;

if (dataCounter>=REQUESTSIZE) break;

}

if (tmpDataCounter<MAXTRAMESIZE-1 && tmpDataCounter<REQUESTSIZE-2) {

gl_tmpBuffer[tmpDataCounter++]=quantityValue;

gl_tmpBuffer[tmpDataCounter++]=curValue;

}

if (tmpDataCounter>=MAXTRAMESIZE) {

for(dataCounter=0;dataCounter<REQUESTSIZE;dataCounter++) gl_request[dataCounter]=0;

return(0); // We loose the trame, because we can't compress it, should never happen.....

}

curValue=gl_request[dataCounter];

quantityValue=0;

}

for(dataCounter=0;dataCounter<tmpDataCounter;dataCounter++) gl_request[dataCounter]=gl_tmpBuffer[dataCounter];

for(;dataCounter<REQUESTSIZE;dataCounter++) gl_request[dataCounter]=0;

return(tmpDataCounter);

}

## Automation management

The system is automated using a language that allows commands to be given or actions to be programmed on receipt of events. Syntax is parsed using a parser() function, and semantics and automation management are handled by a function for managing the system's various requests (from the RS232 link or the CAN bus).

A main loop in the main() function analyzes data from the RS232 link, CAN bus, internal timers, GPIOs or vehicle presence detection on the tracks.

## Software design

## Protocol on serial link

The protocol describes all the data exchanged between a PC and a master board. All data exchanged on the serial bus are in ASCII format, so the value 25 is transmitted with the letter "2" followed by the letter "5". This allows the board to be controlled directly from a terminal. Each transmission could be followed by one or more return messages.

- If message sent to the master needs a specific response

  - a status of a board (DCC or ANA)

  - a list of automation (value and name)

  - a status of a track

  - a status of GPIO

  - a status of a board

- Error: the board number with an error message

## BNF Grammar

Sending a command on the serial bus uses the following BNF grammar:

Command 	::= <Mode> <Board Number> <Control> |

<Global Command>

Fashion		::= PROG | COM

Board Number	::= <between 0 and 31>

Control   	::= <Program> | <Command>

Global Command	::= STOP | RUNALL | RUN <Board Number> |RESET <Board Number>

For Programing Mode (P)

Program		::= <Board Mode> |

<GPIO Setting> |

<Automation> |

<Del Automation>

Board Mode	::= DCC | ANA

GPIO Setting	::= GPIO <GPIO Number> <GPIO Dir>

GPIO number 	::= 0 | 1 | 2 | 3

GPIO Dir	::= IN | OUT

Automation	::= AUT <Identifier> <Manual Status> <Event> ACT <Action>

Manual Status	::= AUTOFF | AUTON

Identify	::= <List of 2 characters max between 2 spaces>

Event		::= BOARD < Board Number> TIMER <Timer Number>

BOARD < Board Number> GPIO < Board Number> <GPIO Level>|

BOARD < Board Number> TRACK < Track Number> STA <vehicle status>

Board Number	::= <between 0 and 31>

Timer number	::= <A value between 0 and 15>

GPIO Level	::= <0 or 1 for output, a value between 0 and 255 and input, counter mode>

Vehicle Status	::= ONTRACK | OFFTRACK

Action		::= TIMER <Timer Number> <Timer Delay>

GPIO <GPIO Number> <GPIO Level> |

LPO <LPO Number> <LPO Level> |

TRACK <Track Number> SPEED <Track Speed> <Track Dir> INERTIA <Inertia>|

DCC <DCC NMRA>|

MANUAL | AUTOMATIC |

AUTOFF < identify> | AUTON < identify>

Timer delay ::= <A value between 0 and 255>

LPO Number	::= 0 | 1 | 2 | 3 | 4 | 5

LPO Level	::= 0 | 1 | 2

Track Number	::= 0 | 1 | 2 | 3 | 4

Track Speed	::= <A value between 0 and 99> | KNOB0 | KNOB1

Track Dir	::= FORW | BACK

Inertia		::= <A value between 0 and 99> | KNOB0 | KNOB1

Del Automation	::= DEL <Automation Number>

For Command Mode (C)

Command		::= < > Action | <GPIO Status> | <LPO Status> | <Track Status> |

<Board Status> |<Automation List> | <Dump Memory> | <Knob Calib>

Action ::= TIMER<Timer Number> <Timer delay>

GPIO <GPIO Number> <GPIO Level> |

LPO <LPO Number> <LPO Level> |

TRACK <Track Number> SPEED <Track Speed> <Track Dir> INERTIA <Inertia>|

DCC <DCC NMRA> |

MANUAL | AUTOMATIC

AUTOFF < identify> | AUTON < identify>

Timer number	::= <A value between 0 and 15>

Timer delay ::= <A value between 0 and 255>

GPIO Number 	::= 0 | 1 | 2 | 3

GPIO Level	::= <0 or 1 for output, a value between 0 and 255 and input, counter mode>

LPO Number	::= 0 | 1 | 2 | 3 | 4 | 5

LPO Level	::= 0 | 1 | 2

Track Number	::= 0 | 1 | 2 | 3 | 4

Track Speed	::= <A value between 0 and 99> | KNOB0 | KNOB1

Inertia		::= <A value between 0 and 99> | KNOB0 | KNOB1

AFC NMRA	::= <See NMRA standard specification>

GPIO Status	::= GSTAT

LPO Status	::= LSTAT

Track Status	::= TSTAT

Board Status ::= BSTAT

Automation List::= AUTLIST

Dump Memory	::= DUMP

Knob Calib	::= CALIB

Any other syntax in command or programing mode should produce an error.

Response from the master

Response		::= <Acknowledge> |

Board <Board Number> : GPIO <GPIO Number> VAL <GPIO Level> <GPIO Dir> |

Board <Board Number> : LPO <LPO Number> VAL <LPO Level> |

Board <Board Number> : KNOB0 VAL <knob 0 value> |

Board <Board Number> : KNOB1 VAL <knob 1 value> |

Board <Board Number> : TRACK <Track Number> SPEED <Track Speed> <Track Dir><vehicle status>

Board <Board Number> : DCC |

Board <Board Number> : ANA |

Board <Board Number>: AUT <Number> <Identifier> ON | AUT <Number> <Identifier> OFF |

Board <Board Number> : <Memory content>

Memory content	::= (<Automation number>,<Data index>,<Data>)<Memory Content>| <empty>

Vehicle Status	::= ONTRACK | OFFTRACK

Acknowledge	::= Board <Board Number> : <Error Message>

## Protocol on CAN bus

Each data exchange on the CAN bus is made up of a frame containing the sender identifier (address value defined with the switch) and a set of data grouped on 8 bytes (Data Field); the other frame information is of no importance here. These fields are described below:

SOF (START OF FRAME): 1 BIT.

Frame start field always equal to 0.

IDENTIFIER: 11-BIT.

Identifies the message sender.

RTR (REMOTE TRANSMISSION REQUEST): 1 BIT.

Usually 0 except in the case of a request frame.

COMMAND: 6-BIT.

Contains the DLC (Data Length Code), the length of data transmitted in of bytes. For example, if 4 bytes of data: DLC = 001000.

DATA : FROM 0 TO 8 BYTES.

CRC (CYCLIC REDUNDANCY CHECK): 16-BIT.

A calculation algorithm is used to check for transmission errors.

ACK (ACKNOWLEDGE): 2 BITS.

Acknowledges whether the frame has been read by a node.

EOF (END OF FRAME): 7 BITS.

Indicates the end of message transmission, 7 bits to 1: 1111111.

The data exchanged on the CAN bus is either text between two 8-byte frames (header 0xCC and footer 0xDD) or compressed data between two 8-byte frames (header 0xEE and footer 0xFF).

## Output display and knob controls

We can connect 2 knobs and an output display of 6 7-segments LEDS compliant with TM1637 protocol on track status GPIO

- GPIO 4 is used for KNOB0 where a value between 0 and 5V must be read.

- GPIO 3 is used for KNOB1 where a value between 0 and 5V must be read.

- GPIO 2 is used for CLK (or SCK) line.

- GPIO 1 is used for DIO (or SDA) line.

On the SPEED and INERTIA command or program, the numerical value can be replaced by KNOB0 or KNOB1, in which case the value read when the command is executed on GPIO 4 or 3 is used.

KNOB0 has a value between -15 and 15, where -15 is 0V and 15 is 5V

KNOB1 has a value between 0 and 100, where 0 is 0V and 100 is 5V

Negative value used on SPEED reverses the running direction. The lowest inertia value corresponds to the shortest time needed to change speed.

Warning: A FORW command with a negative value will result in a BACK side and a BACK command with a negative value will result in a FORW command

The corresponding values are sent to the 7-segments LEDS display

Figure 18 Led Display on TCO

The left side shows the value of KNOB0 and the right side of KNOB1

In MANUAL mode, speed and inertia have an immediate action on all the tracks

.

To calibrate the knobs, turn the min to max values of the two knobs and run the CALIB command. Calibration is saved in EEPROM. Need to be done once after firmware update.

# Command

### Programming DCC mode on a board

| Description | This command allows you to configure a board to operate at the NMRA DCC standard. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> DCC |
| Response | No response |
| Example | To configure board number 5 in DCC mode: PROG 5 DCC |

### Programming ANA mode on a board

| Description | This command allows you to configure a board to operate in analogic mode using PWM on track for speed setting. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> ANA |
| Response | No response |
| Example | To configure board number 5 in ANA mode: PROG 5 ANA |

### Programming AUTOMATIC mode on a board

| Description | This command reactivates all AUTOFF-type commands and disables track control from the speed and inertia knobs. |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> <Event> ACT AUTOMATIC |
| Response | No response |
| Example | To configure board number 2 in AUTOMATIC mode based on GPIO 3 status PROG 2 AUT MC AUTON BOARD 2 GPIO 3 0 ACT AUTOMATIC |

### Programming MANUAL mode on a board

| Description | This command deactivates all AUTOFF-type commands and enables track control from the speed and inertia knobs. MANUAL0 also allows controlling only channel 0; otherwise, all four channels will be controlled by the knobs in this mode. |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> <Event> ACT MANUAL |
| Response | No response |
| Example | To configure board number 2 in MANUAL mode based on GPIO 3 status PROG 2 AUT MC AUTON BOARD 2 GPIO 3 0 ACT MANUAL |

### Initialize a GPIO as output on a board

| Description | This command allows you to configure a GPIO in output mode. This action remains memorized when the power is off (By default, the 4 GPIO are in input mode). Default value in output mode is 1. |
| --- | --- |
| Syntax | PROG <Board Number> GPIO <GPIO Number> OUT |
| Response | No response |
| Example | To configure GPIO 3 on board 5 in output mode PROG 5 GPIO 3 OUT |

### Initialize a GPIO as input on a board

| Description | This command allows you to configure a GPIO in input mode (default mode). This action remains memorized when the power is off In input mode, the GPIO counts level changes from 0 to 255. To reset this counter to 0, simply initialize the GPIO to 0 as if it were an output GPIO, and the counter will be reset to zero. |
| --- | --- |
| Syntax | PROG <Board Number> GPIO <GPIO Number> IN |
| Response | No response |
| Example | To configure GPIO 3 on board 5 in input mode PROG 5 GPIO 3 IN |

### Program an automatic action on a GPIO driven by a timer

| Description | This command allows you to program an automatic change on GPIO output level when a timer is triggered. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TIMER <Timer Number> ACT GPIO <GPIO Number> <GPIO Level> |
| Response | No response |
| Example | To automatically switch the level of GPIO 3 of board 5 to level 1 when the TIMER 2 of board 2 is triggered PROG 5 AUT Test AUTOFF BOARD 2 TIMER 2 ACT GPIO 3 1 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 TIMER 2 => when TIMER 2 of board 2 is triggered. This action is not active in manual mode ACT GPIO 3 1 => GPIO 3 of board 5 is set to 1 |

### Program an automatic action on a LPO driven by a timer

| Description | This command allows you to program an automatic change on LPO output level when a timer is triggered. This action remains memorized when the power is off. (LPO value 2 is used for automatic flashing). |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TIMER <Timer Number> ACT LPO <LPO Number > <LPO Level> |
| Response | No response |
| Example | To automatically switch the level of GPIO 3 of board 5 to level 1 when the TIMER 2 of board 2 is triggered PROG 5 AUT Test AUTON BOARD 2 TIMER 2 ACT LPO 3 1 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTON BOARD 2 TIMER 2 => when TIMER 2 of board 2 is triggered. This action is active in manual mode ACT LPO 3 1 => LPO 3 of board 5 is set to 1 |

### Program an automatic action on a track driven by a timer

| Description | This command allows you to program an automatic change of track speed and direction when a timer is triggered. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TIMER <TIMER Number> ACT TRACK <Track Number> SPEED <Track Speed> <Track Dir> INERTIA <Inertia> |
| Response | No response |
| Example | To slowly change speed and direction of track 2 of board 5 to speed 10 and travel forward when the TIMER 2 of board 2 is triggered PROG 5 AUT Test1 AUTOFF BOARD 2 TIMER 2 ACT TRACK 2 SPEED 10 FORW INERTIA 15 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 TIMER 2 => when TIMER 2 of board 2 is triggered. This action is not active in manual mode ACT TRACK 2 SPEED 10 FORW INERTIA 15 => set track 2 for board 5 at speed 10 forward, 15 steps inertia to change speed |
| How | Speed and/or Inertia value(s) could be replaced by KNOB0 or KNOB1, in this case the value of the knob is read when the action is performed |

### Program on a timer driven by a timer

| Description | This command allows you to program a timer when a timer (the same or another) is triggered. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TIMER <TIMER Number> ACT TIMER <TIMER Number> <TIMER delay> |
| Response | No response |
| Example | To set TIMER 3 on board 5 with a delay of 100 when the TIMER 2 of board 2 is triggered PROG 5 AUT Test AUTOFF BOARD 2 TIMER 2 ACT TIMER 3 100 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 TIMER 2 => when TIMER 2 of board 2 is triggered. This action is not active in manual mode ACT TIMER 3 100 => TIMER 3 is set up at value 100 |

### Turn On or Off an automation driven by a timer

| Description | This command allows you to turn on or off an automation when a timer (the same or another) is triggered. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TIMER <TIMER Number> ACT AUTOFF <Identifier> PROG <Board Number> AUT <Identifier> BOARD <Board Number> TIMER <TIMER Number> ACT AUTON <Identifier> |
| Response | No response |
| Example | To turn on automation Test1 on board 5 when the TIMER 2 of board 2 is triggered PROG 5 AUT Test AUTOFF BOARD 2 TIMER 2 ACT AUTON Test1 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 TIMER 2 => when TIMER 2 of board 2 is triggered. This action is not active in manual mode ACT AUTON Test1 => turn on Test1 |

### Program an automatic action on a GPIO driven by a change of level on a GPIO

| Description | This command allows you to program an automatic change on GPIO output level when the level of a GPIO input has changed on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT GPIO <GPIO Number> <GPIO Level> |
| Response | No response |
| Example | To automatically switch the level of GPIO 3 of board 5 to level 1 when the GPIO 2 of board 2 changes to level 0 PROG 5 AUT Test AUTOFF BOARD 2 GPIO 2 0 ACT GPIO 3 VAL 1 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 GPIO 2 0 => when GPIO 2 of board 2 is set to 0. This action is not active in manual mode ACT GPIO 3 1 => GPIO 3 of board 5 is set to 1 |

### Program an automatic action on a LPO driven by a change of level on a GPIO

| Description | This command allows you to program an automatic change on LPO output level when the level of a GPIO input has changed on a board. This action remains memorized when the power is off. (LPO value 2 is used for automatic flashing). |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT LPO <LPO Number> VAL <LPO Level> |
| Response | No response |
| Example | To automatically switch the level of LPO 3 of board 5 to level 1 when the GPIO 1 of board 2 changes to level 0 PROG 5 AUT Test AUTOFF BOARD 2 GPIO 1 0 ACT LPO 3 VAL 1 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 GPIO 1 VAL 0 => when GPIO 1 of board 2 is set to 0. This action is not active in manual mode ACT LPO 3 1 => LPO 3 of board 5 is set to 1 |

### Program an automatic action on a track driven by a level change on a GPIO

| Description | This command allows you to program an automatic change of track speed and direction when the level of a GPIO input has changed on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT TRACK <Track Number> SPEED <Track Speed> <Track Dir> INERTIA <Inertia> |
| Response | No response |
| Example | To quickly change speed and direction of track 2 of board 5 to speed 10 and travel forward when the GPIO 1 of board 2 changes to level 0 and reverse when the GPIO 1 of board 2 changes to level 1 PROG 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 ACT TRACK 2 SPEED 10 FORW INERTIA 0 PROG 5 AUT Test2 AUTOFF BOARD 2 GPIO 1 1 ACT TRACK 2 SPEED 10 BACK INERTIA 0 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 => Automation Test1 when GPIO 1 of board 2 is set to 0. Not active in manual mode AUT Test2 AUTOFF BOARD 2 GPIO 1 1 => Automation Test2 when GPIO 1 of board 2 is set to 1. Not active in manual mode ACT TRACK 2 SPEED 10 FORW INERTIA 0 => set track 2 for board 5 at speed 10 forward, immediate change ACT TRACK 2 SPEED 10 BACK INERTIA 0 => set track 2 for board 5 at speed 10 backward, immediate change |

### Program an automatic action on a loco (DCC) driven by a level change on a GPIO

| Description | This command allows you to program an automatic change of loco setting when the level of a GPIO input has changed on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT DCC <DCC Standard Command> |
| Response | No response |
| Example | To speed up loco 3 at speed "0xF" forward on board 8 when the GPIO 1 of board 2 changes to level 0 PROG 8 AUT Test AUTOFF BOARD 2 GPIO 1 0 ACT DCC 0x03 0x6F |
| Detail | PROG 8 => programming board number 8 AUT Test AUTOFF BOARD 2 GPIO 1 0 => Automation Test when GPIO 1 of board 2 is set to 0. Not active in manual mode. ACT DCC 0x03 0x6F => to speed up loco "0x3" at speed "0xF" forward |

### Program on a timer driven by a level change on a GPIO

| Description | This command allows you to program a timer when the level of a GPIO input has changed on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT TIMER <TIMER Number> <TIMER delay> |
| Response | No response |
| Example | To set TIMER 3 on board 5 with a delay of 100 when the GPIO 1 of board 2 changes to level 0 PROG 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 ACT TIMER 3 VAL 100 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 => Automation Test1 when GPIO 1 of board 2 is set to 0. Not active in manual mode. ACT TIMER 3 100 => TIMER 3 is set up at value 100 |

### Turn on or off an automation by a level change on a GPIO

| Description | This command allows you to turn on or off an automation when the level of a GPIO input has changed on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT AUTOFF <Identifier> PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> GPIO <GPIO Number> <GPIO Level> ACT AUTON <Identifier> |
| Response | No response |
| Example | To turn off Test2 when the GPIO 1 of board 2 changes to level 0 PROG 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 ACT AUTOFF Test2 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 GPIO 1 0 => Automation Test1 when GPIO 1 of board 2 is set to 0. Not active in manual mode ACT AUTOFF Test2 => turn off Test2 |

### Program an automatic action on a GPIO driven by a change of vehicle presence on a track

| Description | This command allows you to program an automatic change on GPIO output when the presence of a vehicle has changed on a track. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT GPIO <GPIO Number> <GPIO Level> |
| Response | No response |
| Example | To switch the level of GPIO 3 of board 5 to 1 when a vehicle is on track 1 of board 2 and to 0 when a vehicle is no more on track 1 of board 2 PROG 5 AUT Test1 AUTOFF BOARD 2 TRACK 1 STA ONTRACK ACT GPIO 3 1 PROG 5 AUT Test2 AUTOFF BOARD 2 TRACK 1 STA OFFTRACK ACT GPIO 3 0 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test1 when a vehicle is on track 1 of board 2. Not active in manual mode AUT Test2 AUTOFF BOARD 2 TRACK 1 STA OFFTRACK => Automation Test2 when a vehicle is no more on track 1 of board 2. Not active in manual mode ACT GPIO 3 1 => GPIO 3 of board 5 is set to 1 |

### Program an automatic action on a LPO driven by a change of vehicle presence on a track

| Description | This command allows you to program an automatic change on LPO output when the presence of a vehicle has changed on a track. This action remains memorized when the power is off. (LPO value 2 is used for automatic flashing). |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT LPO <LPO Number> <LPO Level> |
| Response | No response |
| Example | To switch the level of LPO 3 of board 5 to 1 when a vehicle is on track 1 of board 2 and to 0 when a vehicle is no more on track 1 of board 2 PROG 5 Test1 AUTOFF AUT BOARD 2 TRACK 1 STA ONTRACK ACT LPO 3 1 PROG 5 Test2 AUTOFF AUT BOARD 2 TRACK 1 STA OFFTRACK ACT LPO 3 0 |
| Detail | PROG 5 => programming board number 5 AUT Test1 AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test1 when a vehicle is on track 1 of board 2. Not active in manual mode AUT Test2 AUTOFF BOARD 2 TRACK 1 STA OFFTRACK => Automation Test2 when a vehicle is no more on track 1 of board 2. Not active in manual mode ACT LPO 3 1 => LPO 3 of board 5 is set to 1 ACT LPO 3 0 => LPO 3 of board 5 is set to 0 |

### Program an automatic action on a track driven by a change of vehicle presence on a track

| Description | This command allows you to program an automatic change of track, speed and direction when the presence of a vehicle has changed on a track. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT TRACK <Track Number> SPEED <Track Speed> <Track Direction> INERTIA <Inertia> |
| Response | No response |
| Example | To change the speed and direction of track 3 to 10 and forward when a vehicle is on track 1 of board 2 PROG 5 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK ACT TRACK 3 SPEED 10 DIR FORW INERTIA 5 |
| Detail | PROG 5 => programming board number 5 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test when a vehicle is on track 1 of board 2. Not active in manual mode ACT TRACK 3 SPEED 10 FORW INERTIA 5=> set speed of track 3 to 10 forward, 5 steps inertia to change speed |
| How | Speed and/or Inertia value(s) could be replaced by KNOB0 or KNOB1, in this case the value of the knob is read when the action is performed |

### Program an automatic action on a loco (DCC) by a change of vehicle presence on a track

| Description | This command allows you to program an automatic change of loco setting when the presence of a vehicle has changed on a track. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT DCC <DCC Standard Command> |
| Response | No response |
| Example | To speed up loco 3 at speed "0xF" forward on board 8 when a vehicle is on track 1 of board 2 PROG 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK ACT DCC 0x03 0x6F |
| Detail | PROG 8 => programming board number 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test when a vehicle is on track 1 of board 2. Not active in manual mode ACT DCC 0x03 0x6F => to speed up loco "0x3" at speed "0xF" forward |

### Program on a timer driven by a change of vehicle presence on a track

| Description | This command allows you to program a timer when the presence of a vehicle has changed on a track. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT TIMER <TIMER Number> <TIMER delay> |
| Response | No response |
| Example | To set TIMER 3 on board 5 with a delay of 100 when a vehicle is on track 1 of board 2 PROG 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK ACT TIMER 3 100 |
| Detail | PROG 8 => programming board number 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test when a vehicle is on track 1 of board 2. Not active in manual mode ACT TIMER 3 100 => TIMER 3 is set up at value 100 |

### Turn on or off an automation by a change of vehicle presence on a track

| Description | This command allows you to turn on or off an automation when the presence of a vehicle has changed on a track. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT AUTOFF <Identifier> PROG <Board Number> AUT <Identifier> <Manual Status> BOARD <Board Number> TRACK <Track Number> STA <Vehicle Status> ACT AUTON <Identifier> |
| Response | No response |
| Example | To turn off Test1 when a vehicle is on track 1 of board 2 PROG 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK ACT AUTOFF Test1 |
| Detail | PROG 8 => programming board number 8 AUT Test AUTOFF BOARD 2 TRACK 1 STA ONTRACK => Automation Test when a vehicle is on track 1 of board 2. Not active in manual mode ACT AUTOFF Test1 => Turn off Test1 |

### Delete an automation on a board

| Description | This command allows you to remove an automation on a board. This action remains memorized when the power is off |
| --- | --- |
| Syntax | PROG <Board Number> DEL <Number> |
| Response | No response |
| Example | To remove automation 3 on board 5 (List of automation could be retrieve using the Automation list command COM <Board Number> AUTLIST) PROG 5 LED 3 |
| Warning | Deleting an automation changes the number of the other automations, so you need to perform once again an AUTLIST command to know the new numbers |

### Force the value on a GPIO on a board

| Description | This command allows you to set the level of a GPIO. This action remains valid until the board is stopped or an automatic action is triggered. When GPIO is in input mode, this command reset the counter when set to 0 |
| --- | --- |
| Syntax | COM <Board Number> GPIO <GPIO Number> <GPIO Level> |
| Response if GPIO in output mode | No response |
| Example | To set GPIO 3 at level 1 on board 4 COM 4 GPIO 3 1 |

### Create a timer on a board

| Description | This command allows you to set a timer. This action remains valid until the board is stopped or the timer is triggered |
| --- | --- |
| Syntax | COM <Board Number> TIMER <Timer Number> <Timer delay> |
| Response if GPIO in output mode | No response |
| Example | To set TIMER 3 with a delay of 10 on board 4 COM 4 TIMER 3 10 |

### Force the value on a LPO on a board

| Description |  | This command allows you to set the level of a LPO. This action remains valid until the board is stopped or an automatic action is triggered. (LPO value 2 is used for automatic flashing). |
| --- | --- | --- |
| Syntax |  | COM <Board Number> LPO <LPO Number> <LPO Level> |
| Response |  | No response |
| Example |  | To set LPO 3 at level 1 on board 4 COM 4 LPO 3 1 |

### Turn on an automation on a board

| Description |  | This command allows you to turn on an automation. This action remains valid until the board is stopped or an automatic action is triggered |
| --- | --- | --- |
| Syntax |  | COM <Board Number> AUTON <Identifier> |
| Response |  | No response |
| Example |  | To turn on automation Test On board 4 COM 4 AUTON Test |

### Turn off an automation on a board

| Description |  | This command allows you to turn off an automation. This action remains valid until the board is stopped or an automatic action is triggered |
| --- | --- | --- |
| Syntax |  | COM <Board Number> AUTOFF <Identifier> |
| Response |  | No response |
| Example |  | To turn off automation Test On board 4 COM 4 AUTOFF Test |

### Force speed and direction on a track

| Description | This command allows you to set the speed and direction on a track. This action remains valid until the board is stopped or an automatic action is triggered |
| --- | --- |
| Syntax | COM <Board Number> TRACK <Track Number> SPEED <Track Speed> <Dir> INERTIA <Inertia> |
| Response | No response |
| Response if board in DCC mode | Board <Board Number> Bad mode |
| Example | To set speed at 4 and direction forward on track 2 board 8 COM 8 TRACK 2 SPEED 4 FORW INERTIA 2 |
| How | Speed and/or Inertia value(s) could be replaced by KNOB0 or KNOB1, in this case the value of the knob is read when the action is performed |

### Send a DCC command to a board

| Description | This command allows you to send a DCC NMRA command to a board (all tracks). This action remains valid until the board is stopped or an automatic action is triggered |
| --- | --- |
| Syntax | COM <Board Number> DCC <DCC Stand Command> |
| Response | No response |
| Response if board in ANA mode | Board <Board Number> Bad mode |
| Example | To send a command to speed up loco 3 at speed "0xF" forward on board 8 COM 8 DCC 0x03 0x6F To send a command to speed up loco 3 at speed "0xF" backward on board 8 COM 8 DCC 0x03 0x4F |

### Request the status of all the GPIOs on a board

| Description | This command allows you to get the direction and the level of all the GPIOs on a board |
| --- | --- |
| Syntax | COM <Board Number> GSTAT |
| Response | Board <Board Number> GPIO 0 <GPIO Dir> VAL <GPIO Level> COUNT <GPIO Counter> Board <Board Number> GPIO 1 <GPIO Dir> VAL <GPIO Level> COUNT <GPIO Counter> Board <Board Number> GPIO 2 <GPIO Dir> VAL <GPIO Level> COUNT <GPIO Counter> Board <Board Number> GPIO 3 <GPIO Dir> VAL <GPIO Level> COUNT <GPIO Counter> Board <Board Number> KNOB0 VAL <knob value> Board <Board Number> KNOB1 VAL <knob value> |
| Example | To get the status of all the GPIO of board 5 COM 5 GSTAT |

### Request the status of all the LPOs on a board

| Description | This command allows you to get the direction and the level of all the LPOs on a board |
| --- | --- |
| Syntax | COM <Board Number> LSTAT |
| Response | Board <Board Number> LPO 0 VAL <LPO Level> Board <Board Number> LPO 1 VAL <LPO Level> Board <Board Number> LPO 2 VAL <LPO Level> Board <Board Number> LPO 3 VAL <LPO Level> Board <Board Number> LPO 4 VAL <LPO Level> Board <Board Number> LPO 5 VAL <LPO Level> |
| Example | To get the status of all the LPO of board 5 COM 5 LSTAT |

### Request track status on a board

| Description | This command allows you to get the direction and the speed and direction of all the tracks on a board if the board is in ANA mode |
| --- | --- |
| Syntax | COM <Board Number> TSTAT |
| Response | Board <Board Number> TRACK 0 SPEED <Track Speed> <Track Dir> <vehicle status> Board <Board Number> TRACK 1 SPEED <Track Speed> <Track Dir> <vehicle status> Board <Board Number> TRACK 2 SPEED <Track Speed> <Track Dir> <vehicle status> Board <Board Number> TRACK 3 SPEED <Track Speed> <Track Dir> <vehicle status> |
| Example | To get the status of all the tracks of board 5 COM 5 TSTAT |

### Request board status

| Description | This command allows you to get the status of a board to know if the running mode is DCC or ANA |
| --- | --- |
| Syntax | COM <Board Number> BSTAT |
| Response if DCC mode | Board <Board Number> DCC |
| Response if ANA mode | Board <Board Number> ANA |
| Example | To get the status of the board 5 COM 5 BSTAT |

### Request the list of actions programmed on a board

| Description | This command allows you to get all the Automations stored in a board |
| --- | --- |
| Syntax | COM <Board Number> AUTLIST |
| Response if no data | No response |
| Response for each automation | Board <Board Number> <Number> <Identifier> ON Board <Board Number> <Number> <Identifier> OFF |
| Example | To get the list of actions programmed on board 5 COM 5 AUTLIST |

### Request a dump of memory on a board

| Description | This command allows you to get the content of Automations stored in a board in binary mode (reserved for PC application) |
| --- | --- |
| Syntax | COM <Board Number> DUMP |
| Response if no data | No response |
| Response for each automation | Board <Board Number> <Number> (<Number>,<Index>,<Data>)... |
| Example | To get the dump of board 5 COM 5 DUMP |

### Request a calibration of knobs

| Description | This command is used to save the min and max values of knobs 0 and 1. To do this, turn both knobs to the min value and then to the max value and execute this command. |
| --- | --- |
| Syntax | CALIB <Board Number> |
| Response | No response |
| Example | To calibrate knobs of board 5 CALIB 5 |

## Global command

### Stop all

| Description | This command will stop the activity on all the boards of the network |
| --- | --- |
| Syntax | STOP |
| Response | No response |
| Example | To stop all STOP |

### Run all

| Description | This command will start the activity on all the boards of the network |
| --- | --- |
| Syntax | RUNALL |
| Response | No response |
| Example | To run all RUNALL |

### Run a specific board

| Description | This command will start the activity on a specific board |
| --- | --- |
| Syntax | RUN <Board Number> |
| Response | No response |
| Example | To run board 5 RUN 5 |

### Reset all automation of a specific board

| Description | This command will delete all the automation on a specific board. Warning this action is performed immediately without any notification! |
| --- | --- |
| Syntax | RESET <Board Number> |
| Response | No response |
| Example | To reset board 5 RESET 5 |

### Inconsistent programming

If an impossible automatic action is triggered, such as setting the speed and direction on the track of a board in DCC mode, or configuring a locomotive on a map in ANA mode, the action is simply ignored and a message is sent to the serial console:

- Unknown token

- Number missing

- Incomplete request

- Bad number

- Mode is missing

- Bad GPIO number

- Bad TIMER number

- Bad TIMER value

- Bad LPO number

- Bad Automation status

- Bad Automation name

- Automation name missing

- Bad GPIO direction

- Bad GPIO level

- Bad Low Power Output level

- Bad track speed

- Bad track direction

- Bad track number

- Bad fashion

- Wrong board number

- No more automation available

- Space missing

- Identifier is too long

- Automation already defined

- Wrong automation number

- Bad inertia value

- Bad user mode

# Barraux's railway network (31/07/2025)

### General System Structure

- The system uses several boards (from board 0 to board 3, plus a special board "MASTER 31").

- Each board controls:

  - Up to 4 tracks (tracks A0 to D3 for board 31, E0 to H3 for board 0, etc.)

  - 6 low-power outputs (LPO): for LEDs, lights, switch controls, etc.

  - 4 GPIO inputs: for buttons, optical sensors, contacts, etc.

  - 4 "Track Status" inputs: for 7-segment displays, speed and inertia potentiometers.

Figure 19 Railway network (31/07/2025)

### ⚙️ Operating Modes

#### ▶️ Automatic Mode

Commands like:

PROG <board> AUT <ID> AUTON/AUTOFF BOARD <n°> ... ACT ...

- Activates complex sequences with timing (TIMER), train detection (STA ONTRACK), rail actions (TRACK ... SPEED ...), LEDs (LPO ...), and I/O.

- Examples:

  - Launching a train based on a timer.

  - Controlling traffic lights (green/red) as trains pass.

  - Specific sequences for uncoupling wagons, level crossings, reversing locomotives, etc.

#### ⏸️ Manual Mode

Commands like:

PROG <board> AUT M<ID> AUTON BOARD <n°> ... ACT ...

- GPIO resets.

- Stops tracks (speed 0, inertia 0).

- Turns indicator LEDs on/off (e.g., start button LED, manual mode indicator).

### 🧠 Automation per Board

#### 🟩 MASTER 31 Board

- Controls tracks A0 to D3.

- Coordinates all other boards.

- Includes an optical control panel (TCO).

- Automations: loop sequences with station wait times, signal light control, train restart, synchronized timers (TIMER 1, TIMER 2, etc.).

#### 🟦 Board 0

- Controls tracks E0 to H3.

- Includes logic for uncoupling and coordination with board 1 (rack train).

- Advanced management of two trains coexisting on internal tracks.

#### 🟨 Board 1

- Controls tracks I0 to L3.

- Detailed sequence for fully automatic locomotive reversal.

- Uses six timers (TIMER 1 to TIMER 6).

- Tightly coupled with boards 0 and 2.

#### 🟧 Board 2

- Controls one tramway track (M0) and two temporary Faller AMS tracks.

- Extensive commands for level crossing lights, optical sensors, and regular back-and-forth sequences.

- Manages TCO LEDs to display tramway position.

#### 🟥 Board 3

- Dedicated board for manual/automatic switch control.

- Examples: toggling between main and passing track, dual-press manual commands.

### 📌 TCO (Optical Control Panel) Commands

- Enable manual or automatic control of events like:

  - Triggering red lights.

  - Switches when a train arrives or departs.

  - Activating or stopping uncoupling actions.

- Prefixes:

  - M<i>: manual mode.

  - A<i>: automatic.

  - T<i>: TCO command.

  - D<i>: deactivatable (can be temporarily removed from execution logic).

### 📌 Programing the boards

A Qt application is available to send the programming code to the different boards via board 31. This application recognizes the boards' programming language, can check the syntax before sending the instructions, and includes a graphical visualization of the interconnections between the commands.

The Qt code is available in RailManagerUI, a customized version of the open-source project Scribe, described as:

A simple, user-friendly text editor.

GitHub Stars: https://github.com/AleksandrHovhannisyan/Scribe-Text-Editor/stargazers
Latest Release: https://github.com/AleksandrHovhannisyan/Scribe-Text-Editor/releases

This application also incorporates the QtNodes library, which has been modified to meet the specific needs of the project.

As of July 31, 2025, the application is still under development but already functional. It is configured for compilation on both macOS and Windows platforms.

### Source Code and Project Access

The code, along with the entire development of this railway network, is available on the public GitHub repository:

https://github.com/PhilippeCavenel/RAIL-DRIVER

Figure 20 Rail Manager UI, sribe interface

Figure 21 Rail Manager UI, node interface

By clicking on a node, you can easily view the interconnections.

Figure 22 Rail Manager UI, node highlighted

### Board Code (27/08/2025)

//================================================

// VERSION DU 21 AOUT 2025

//================================================

// Gestion des numeros de piste

// Entre 0 et 3, numéro de piste unique

// Entre 17 et 31 :

// 17 = 0

// 18 = 1

// 19 = 1 0

// 20 = 2

// 21 = 2 0

// 22 = 2 1

// 23 = 2 1 0

// 24 = 3

// 25 = 3 0

// 26 = 3 1

// 27 = 3 1 0

// 28 = 3 2

// 29 = 3 2 0

// 30 = 3 2 1

// 31 = 3 2 1 0

//================================================

//================================================

// GESTION DES AUTOMATISMES DE LA CARTE MASTER 31

//================================================

// Connect Rail 0	Voie A0

// Connect Rail 1	Voie B1

// Connect Rail 2	Voie C2

// Connect Rail 3	Voie D3

// LPO 0		Feu vert B1

// LPO 1		Feu rouge B1

// LPO 2		Led bouton bleu

// LPO 3		Led mode manuel

// LPO 4		décrochage wagon circuit bas

// LPO 5		non connecté

// GPIO input 0		Bouton aiguillage CROSS rouge

// GPIO input 1		Bouton bleu démarrer

// GPIO input 2		Bouton aiguillage CROSS noir

// GPIO input 3		Contact mode manuel

// Track Status	0	Afficheur 7 segments DIO

// Track Status	1	Afficheur 7 segments CLK

// Track Status	2	Potentiomètre inertie

// Track Status	3	Potentiomètre vitesse

//================================================

// GESTION DES AUTOMATISMES DE LA CARTE 0

//================================================

// Connect Rail 0	Voie E0

// Connect Rail 1	Voie F1

// Connect Rail 2	Voie G2

// Connect Rail 3	Voie H3

// LPO 0		Feu vert E0

// LPO 1		Feu rouge E0

// LPO 2		Led bouton blanc

// LPO 3		Led mode manuel

// LPO 4		décrochage gare crémaillère haut

// LPO 5		voie CROSS sur voie F (0) / voie CROSS non connecté

// GPIO input 0		Bouton aiguillage D3 / H3 et C2/ G2 rouge

// GPIO input 1		Bouton blanc démarrer

// GPIO input 2		Bouton aiguillage D3 / H3 et C2/ G2 noir

// GPIO input 3		Contact mode manuel

// Track Status	0	Afficheur 7 segments DIO

// Track Status	1	Afficheur 7 segments CLK

// Track Status	2	Potentiomètre inertie

// Track Status	3	Potentiomètre vitesse

//================================================

// GESTION DES AUTOMATISMES DE LA CARTE 1

//================================================

// Connect Rail 0	Voie I0

// Connect Rail 1	Voie J1

// Connect Rail 2	Voie K2

// Connect Rail 3	Voie L3

// LPO 0		aiguillage E0 / (E0) H1 et E0 / G2

// LPO 1		non connecté (relié à un relai)

// LPO 2		Led bouton jaune

// LPO 3		Led mode manuel

// LPO 4		aiguillage I0 / J1 , K2

// LPO 5		non connecté (relié à un relai)

// GPIO input 0		aiguillage E0 / (E0) H1 et E0 / G2

// GPIO input 1		Bouton Jaune

// GPIO input 2		aiguillage E0 / (E0) H1 et E0 / G2

// GPIO input 3		Contact mode manuel

// Track Status	0	Afficheur 7 segments DIO

// Track Status	1	Afficheur 7 segments CLK

// Track Status	2	Potentiomètre inertie

// Track Status	3	Potentiomètre vitesse

//================================================

// GESTION DES AUTOMATISMES DE LA CARTE 2

//================================================

// Connect Rail 0	Voie M0

// Connect Rail 1	Voie voiture 1 - temporaire en attendant d'avoir d'autres cartes

// Connect Rail 2	Voie voiture 2 - temporaire en attendant d'avoir d'autres cartes

// Connect Rail 3	Voie non connectée

// LPO 0		Ouverture barrière passage à niveau

// LPO 1		Fermeture barrière passage à niveau

// LPO 2		Led bouton orange

// LPO 3		Led mode manuel

// LPO 4		Led tramway devant grue

// LPO 5		Led tramway devant gare terminale

// GPIO input 0		Détecteur optique

// GPIO input 1		Bouton orange

// GPIO input 2		Détecton passage tramway devant capteur magnétique

// GPIO input 3		Contact mode manuel

// Track Status	0	Afficheur 7 segments DIO

// Track Status	1	Afficheur 7 segments CLK

// Track Status	2	Potentiomètre inertie

// Track Status	3	Potentiomètre vitesse

//================================================

// GESTION DES AUTOMATISMES DE LA CARTE 3

//================================================

// Connect Rail 0	Voie non connectée

// Connect Rail 1	Voie non connectée

// Connect Rail 2	Voie non connectée

// Connect Rail 3	Voie non connectée

// LPO 0		aiguillage CROSS noir

// LPO 1		aiguillage CROSS rouge

// LPO 2		aiguillage B1 / CROSS (noir cmd manuel double appui)

// LPO 3		aiguillage B1 / CROSS (rouge cmd manuel double appui)

// LPO 4		non connecté (relié à un relai)

// LPO 5		aiguillage D3 / H3 et C2 / G2

// GPIO input 0		aiguillage I0 / J1 , K2

// GPIO input 1		aiguillage I0 / J1 , K2

// GPIO input 2		Commande décrochage wagon gare crémaillère haut

// GPIO input 3		Commande décrochage wagon circuit bas

// Track Status	0	non connecté

// Track Status	1	non connecté

// Track Status	2	non connecté

// Track Status	3	non connecté

//============================================================

// PRINCIPES GENERAUX DE FONCTIONNEMENT POUR TOUTES LES CARTES

//============================================================

// Passage en mode manuel

//	- passe en vitesse nulle toutes les voies de la carte jusqu'à modification d'une valeur de vitesse ou d'internie

//	- allume la led rouge du mode manuel

//	- Eteint la led du bouton demarrer si elle était allumée

//	- Réinitialise tous les TIMERs

//	- Passage en mode manuel et désactivation de toutes les commandes de type AUTOFF

// Passage en mode automatique

//	 - Eteint la led rouge du bouton mode manuel

//	 - Passage en mode automatique

//	 - L'appui sur le bouton démarrer

//		- Allume la led du bouton démarrer en mode clignotement

//		- Lance la séquence automatique programmée

//============================================================

// REGLES DE NOMAGE DES COMMANDES

// ------------------------------

// M<i> => MANUEL

// A<i> => AUTOMATIQUE

// T<i> => Gestion commandes du TCO

// D<i> => Commande pouvant être désactivée (via AUTOFF spécifique non lié au mode AUTOMATIC ou MANUAL)

// Les commandes en mode AUTON sont toujours actives en mode MANUAL et AUTOMATIC sauf si elles ont été désactivées par une commande spécifique AUTOFF

// Les commandes en mode AUTOFF sont désactivées en mode MANUAL

// Au démarrage des cartes, toutes les commandes AUTOFF sont désactivées, il faut donc spécifier au moins une fois un passage en mode AUTOMATIC

//============================================================

// ARRET COMPLET DES AUTOMATISMES

//============================================================

STOPALL

//============================================================

// GESTION DES MODES MANUELS

//============================================================

////////////////////////////////////////////////////////

// CARTE MASTER 31 pour les 4 voies externes A0,B1,C2,D3

////////////////////////////////////////////////////////

// Passe en vitesse nulle sur toutes les voies de la carte

PROG 31 AUT M1 AUTON BOARD 31 GPIO 3 0 ACT TRACK 31 SPEED 0 FORW INERTIA 0

// Allume la led rouge du mode manuel

PROG 31 AUT M2 AUTON BOARD 31 GPIO 3 0 ACT LPO 3 1

// Eteint la led du bouton demarrer

PROG 31 AUT M3 AUTON BOARD 31 GPIO 3 0 ACT LPO 2 0

// Réinitialise GPIO

PROG 31 AUT M4 AUTON BOARD 31 GPIO 3 0 ACT GPIO 3 0

// passage en mode manuel

PROG 31 AUT M5 AUTON BOARD 31 GPIO 3 0 ACT MANUAL

// Réinitialise GPIO

PROG 31 AUT M6 AUTON BOARD 31 GPIO 1 0 ACT GPIO 1 0

////////////////////////////////////////////////

// CARTE 0 pour les 4 voies internes E0,F1,G2,H3

////////////////////////////////////////////////

// Passe en vitesse nulle toutes les voies de la carte

PROG 0 AUT M1 AUTON BOARD 0 GPIO 3 0 ACT TRACK 31 SPEED 0 FORW INERTIA 0

// Allume la led rouge du mode manuel

PROG 0 AUT M2 AUTON BOARD 0 GPIO 3 0 ACT LPO 3 1

// Eteint la led du bouton demarrer

PROG 0 AUT M3 AUTON BOARD 0 GPIO 3 0 ACT LPO 2 0

// Réinitialise GPIO

PROG 0 AUT M4 AUTON BOARD 0 GPIO 3 0 ACT GPIO 3 0

// passage en mode manuel

PROG 0 AUT M5 AUTON BOARD 0 GPIO 3 0 ACT MANUAL

// Réinitialise GPIO

PROG 0 AUT M6 AUTON BOARD 0 GPIO 1 0 ACT GPIO 1 0

/////////////////////////////////////////////////////////

// CARTE 1 pour les 4 voies de la crémaillère I0,J1,K2,L3

/////////////////////////////////////////////////////////

// Passe en vitesse nulle toutes les voies de la carte

PROG 1 AUT M1 AUTON BOARD 1 GPIO 3 0 ACT TRACK 31 SPEED 0 FORW INERTIA 0

// Allume la led rouge du mode manuel

PROG 1 AUT M2 AUTON BOARD 1 GPIO 3 0 ACT LPO 3 1

// Eteint la led du bouton demarrer

PROG 1 AUT M3 AUTON BOARD 1 GPIO 3 0 ACT LPO 2 0

// Réinitialise GPIO

PROG 1 AUT M4 AUTON BOARD 1 GPIO 3 0 ACT GPIO 3 0

// Passage en mode manuel

PROG 1 AUT M5 AUTON BOARD 1 GPIO 3 0 ACT MANUAL

// Réinitialise GPIO

PROG 1 AUT M6 AUTON BOARD 1 GPIO 1 0 ACT GPIO 1 0

///////////////////////////////////////////////////////////////////////

// CARTE 2 pour la voie du tramway et les 2 voies temporaire Faller AMS

///////////////////////////////////////////////////////////////////////

// Passe en vitesse nulle toutes les voies de la carte

PROG 2 AUT M1 AUTON BOARD 2 GPIO 3 0 ACT TRACK 31 SPEED 0 FORW INERTIA 0

// Allume la led rouge du mode manuel

PROG 2 AUT M2 AUTON BOARD 2 GPIO 3 0 ACT LPO 3 1

// Eteint la led du bouton demarrer

PROG 2 AUT M3 AUTON BOARD 2 GPIO 3 0 ACT LPO 2 0

// Réinitialise GPIO

PROG 2 AUT M4 AUTON BOARD 2 GPIO 3 0 ACT GPIO 3 0

// Passage en mode manuel (voie 0)

PROG 2 AUT M5 AUTON BOARD 2 GPIO 3 0 ACT MANUAL0

// Réinitialise GPIO

PROG 2 AUT M6 AUTON BOARD 2 GPIO 1 0 ACT GPIO 1 0

// Eteint affichage des LEDs du TCO (position du Tramway sur la voie)

PROG 2 AUT M7 AUTON BOARD 2 GPIO 3 0 ACT LPO 4 0

PROG 2 AUT M8 AUTON BOARD 2 GPIO 3 0 ACT LPO 5 0

// Ferme les barrières

PROG 2 AUT M9 AUTON BOARD 2 GPIO 3 0 ACT LPO 0 0

PROG 2 AUT MA AUTON BOARD 2 GPIO 3 0 ACT LPO 1 1

//============================================================

// GESTION DES MODES AUTOMATIQUES

//============================================================

/////////////////////////////////////////////////////////

// CARTE MASTER 31 pour les 4 voies externes A0,B1,C2,D3

// TIMER 1 => Temps d'attente en gare

// TIMER 2 => Temps de circulation sur les voies

////////////////////////////////////////////////////////

// Passage en mode automatique

PROG 31 AUT A0 AUTON BOARD 31 GPIO 3 1 ACT AUTOMATIC

// Eteint la led rouge du bouton mode manuel

PROG 31 AUT A1 AUTON BOARD 31 GPIO 3 1 ACT LPO 3 0

// Arme TIMER 1 pour lancer le cycle de démarrage sur les voies A0, B1, C2 et D3 par appui bouton

PROG 31 AUT A2 AUTOFF BOARD 31 GPIO 1 1 ACT TIMER 1 1

// Lance la séquence automatique programmée

// DESCRIPTION DE LA SEQUENCE

// --------------------------

// Sur appui bouton démarrer, le train va effectuer des tours pendant 10 unités de temps TIMER (environ 1mn20),

// puis s'arrêter en gare sur la même période avant de recommencer

// Allume la led du bouton démarrer (clignotement)

PROG 31 AUT A3 AUTOFF BOARD 31  GPIO 1 1  ACT LPO 2 2

// Active le démarrage du train voie A0, B1, C2 et D3 sur appui bouton démarrer

PROG 31 AUT A4 AUTOFF BOARD 31 GPIO 1 1 ACT AUTON D4

// Relance du train après passage du train à crémaillère

PROG 31 AUT A5 AUTOFF BOARD 31 GPIO 3 1 ACT AUTOFF D5

PROG 31 AUT A6 AUTOFF BOARD 31 GPIO 1 1 ACT AUTON D5

// Passage du feu au vert

PROG 31 AUT A7 AUTOFF BOARD 31 TIMER 1 ACT LPO 1 0

PROG 31 AUT A8 AUTOFF BOARD 31 TIMER 1 ACT LPO 0 1

// Arme TIMER 2 pour faire tourner le train pendant 10 unités

PROG 31 AUT A9 AUTOFF BOARD 31 TIMER 1 ACT TIMER 2 10

// Désactive l'arret en gare sur détection de train voie A0

PROG 31 AUT AA AUTOFF BOARD 31 TIMER 1 ACT AUTOFF D1

//Désactive Le passage du feu au rouge sur détection de train voie A0

PROG 31 AUT AB AUTOFF BOARD 31 TIMER 1 ACT AUTOFF D2

PROG 31 AUT AC AUTOFF BOARD 31 TIMER 1 ACT AUTOFF D3

// Réactive l'arret en gare sur déclenchement du TIMER 2

PROG 31 AUT AD AUTOFF BOARD 31 TIMER 2 ACT AUTON D1

// Réactive le passage du feu au rouge sur détection de train voie A0

PROG 31 AUT AE AUTOFF BOARD 31 TIMER 2 ACT AUTON D2

PROG 31 AUT AF AUTOFF BOARD 31 TIMER 2 ACT AUTON D3

// Réarme le TIMER 1 pour une attente en gare de 10 unités

PROG 31 AUT AG AUTOFF BOARD 31 TIMER 2 ACT TIMER 1 10

// Commande pouvant être désactivée

// --------------------------------

// Arret en gare sur détection voie A0, les voies A0, B1, C2 et D3 sont progressivement arrêtées

PROG 31 AUT D1 AUTOFF BOARD 31 TRACK 0 STA ONTRACK ACT TRACK 31 SPEED 0 FORW INERTIA 48

// Passage du feu au rouge sur détection de train voie A0

PROG 31 AUT D2 AUTOFF BOARD 31 TRACK 0 STA ONTRACK ACT LPO 1 1

PROG 31 AUT D3 AUTOFF BOARD 31 TRACK 0 STA ONTRACK ACT LPO 0 0

// Démarre le train voie A0, B1, C2 et D3 sur déclenchement TIMER 1 selon valeurs de consigne du TCO, cette commande peut être désactivée pour le retournement du train à crémaillère

PROG 31 AUT D4 AUTOFF BOARD 31 TIMER 1 ACT TRACK 31 SPEED 9 FORW INERTIA 30

// On relance le train si le démarrage a été activé quand le train à crémaillère arrive en haut

PROG 31 AUT D5 AUTOFF BOARD 1 TRACK 2 STA ONTRACK ACT TIMER 1 1

/////////////////////////////////////////////////////////////////////

// CARTE 0 pour les 4 voies internes E0,F1,G2,H3

//TIMER 1 => mode normal de la voie

//TIMER 2 => Retour de la locomotive pour récupérer les wagons détachés

/////////////////////////////////////////////////////////////////////

// Passage en mode automatique

PROG 0 AUT A0 AUTON BOARD 0 GPIO 3 1 ACT AUTOMATIC

// Eteint la led rouge du bouton mode manuel

PROG 0 AUT A1 AUTON BOARD 0 GPIO 3 1 ACT LPO 3 0

// Mode normal de la carte par appui bouton démarrer ou action automatique du train à crémaillère

PROG 0 AUT A2 AUTOFF BOARD 0 GPIO 1 1 ACT TIMER 1 1

// Allume la led du bouton démarrer (clignotement)

PROG 0 AUT A3 AUTOFF BOARD 0 GPIO 1 1 ACT LPO 2 2

// Lance la séquence automatique programmée

// DESCRIPTION DE LA SEQUENCE

// --------------------------

// Sur appui bouton démarrer, le train va effectuer des tours jusqu'à ce que le train à crémaillère quitte la gare du haut, afin de permettre à ce dernier de faire un retournement de la locomotive sur les voies internes. Le redémarrage du train s'effectuera par déclenchement du Timer 1, activé quand le wagon du train à crémaillère déclenche le capteur optique pour retour vers la gare centrale.

// Démarre le train voie E0, F1, G2 et H3

PROG 0 AUT A5 AUTOFF BOARD 0 TIMER 1 ACT TRACK 31 SPEED 11 FORW INERTIA 56

// Passage du feu au vert

PROG 0 AUT A6 AUTOFF BOARD 0 TIMER 1 ACT LPO 1 0

PROG 0 AUT A7 AUTOFF BOARD 0 TIMER 1 ACT LPO 0 1

// Désactive l'arret en gare sur détection de train voie F1

PROG 0 AUT A8 AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D1

//Désactive Le passage du feu au rouge sur détection de train voie F1

PROG 0 AUT A9 AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D2

PROG 0 AUT AA AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D3

// Désactive la relance du train sur détection voie H3

PROG 0 AUT AB AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D4

// Désactivation des commandes pour le train à crémaillère

PROG 0 AUT AC AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D5

PROG 0 AUT AD AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D6

PROG 0 AUT AE AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D7

PROG 0 AUT AF AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D8

PROG 0 AUT AG AUTOFF BOARD 0 TIMER 1 ACT AUTOFF D9

PROG 0 AUT AH AUTOFF BOARD 0 TIMER 1 ACT AUTOFF DA

PROG 0 AUT AI AUTOFF BOARD 0 TIMER 1 ACT AUTOFF DB

PROG 0 AUT AJ AUTOFF BOARD 0 TIMER 1 ACT AUTOFF DC

PROG 0 AUT AK AUTOFF BOARD 0 TIMER 1 ACT AUTOFF DD

// En mode automatique, même si le démarrage n'est pas lancé, la commande DD ne doit pas être active avant le retour du train à crémaillère

PROG 0 AUT AL AUTOFF BOARD 0 GPIO 3 1 ACT AUTOFF DD

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// Ces différentes commandes sont pilotées depuis la carte 1 Crémaillère au travers du TIMER 6 (lorsque le train à crémaillère repart de la gare du haut)

// Active l'arret en gare sur détection de train voie F1

PROG 0 AUT B0 AUTOFF BOARD 1 TIMER 6 ACT AUTON D1

// Active le passage du feu au rouge sur détection de train voie F1

PROG 0 AUT B1 AUTOFF BOARD 1 TIMER 6 ACT AUTON D2

PROG 0 AUT B2 AUTOFF BOARD 1 TIMER 6 ACT AUTON D3

// Active la relance du train sur détection voie H3

PROG 0 AUT B3 AUTOFF BOARD 1 TIMER 6 ACT AUTON D4

// Désactive la relance du train sur détection voie H3, car le train à crémaillère passe par là

PROG 0 AUT B4 AUTOFF BOARD 1 TIMER 2 ACT AUTOFF D4

// Désactive l'arret du train en gare principale sur détection du train en voie F1, car le train à crémaillère passe par là

PROG 0 AUT B5 AUTOFF BOARD 1 TIMER 2 ACT AUTOFF D1

// Activation des commandes pour le train à crémaillère dès activation du TIMER 2 de la carte 1

PROG 0 AUT B6 AUTOFF BOARD 1 TIMER 2 ACT AUTON D5

PROG 0 AUT B7 AUTOFF BOARD 1 TIMER 2 ACT AUTON D6

PROG 0 AUT B8 AUTOFF BOARD 1 TIMER 2 ACT AUTON D7

PROG 0 AUT B9 AUTOFF BOARD 1 TIMER 2 ACT AUTON D8

PROG 0 AUT BA AUTOFF BOARD 1 TIMER 2 ACT AUTON D9

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// Commande pouvant être désactivée

// --------------------------------

// => Gestion automatique décrochage circuit caché et aiguillage E0 / (E0) H1 et E0 / G2, voir les commandes automatiques du TCO

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// Gestion normale de la carte pour arret du train en gare principale

// Arret en gare, les voies E0, F1, G2 et H3 sont progressivement arrêtées sur détection de train voie F1

PROG 0 AUT D1 AUTOFF BOARD 0 TRACK 1 STA ONTRACK ACT TRACK 31 SPEED 0 FORW INERTIA 60

// Passage du feu au rouge sur détection de train voie F1

PROG 0 AUT D2 AUTOFF BOARD 0 TRACK 1 STA ONTRACK ACT LPO 1 1

PROG 0 AUT D3 AUTOFF BOARD 0 TRACK 1 STA ONTRACK ACT LPO 0 0

// Si le train est détecté sur la voie H3 car il a reçu trop tard la commande d'arrêt, il faut le remettre en route pour un tour

PROG 0 AUT D4 AUTOFF BOARD 0 TRACK 3 STA ONTRACK ACT TRACK 31 SPEED 11 FORW INERTIA 56

/////////////////////////////////////////////////////////////////////////////////////////////////////////

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// Gestion de la carte pour le train à crémaillère

// Démarrage train à crémaillère vers décrochage partie cachée

PROG 0 AUT D5 AUTOFF BOARD 1 TIMER 2 ACT TRACK 30 SPEED 7  BACK INERTIA 15

// Pas de passage de train sur la voie externe quand on retourne la locomotive, donc on bloque le train en gare

PROG 31 AUT AH AUTOFF BOARD 1 TIMER 2 ACT AUTON D1

PROG 31 AUT AI AUTOFF BOARD 1 TIMER 2 ACT AUTON D2

PROG 31 AUT AJ AUTOFF BOARD 1 TIMER 2 ACT AUTON D3

PROG 31 AUT AK AUTOFF BOARD 1 TIMER 2 ACT AUTOFF D4

// ON désactive le blocage du train en gare quand le train à crémaillère arrive en haut

PROG 31 AUT AL AUTOFF BOARD 1 TRACK 2 STA ONTRACK ACT AUTON D4

// Activation de la fonction de réinitialisation du capteur optique lorsque la locomotive arrive sur la piste H3

PROG 2 AUT E0 AUTON BOARD 1 TIMER 2 ACT AUTON E1

// Lorsque le train à crémaillère arrive sur la piste 3 on active le décrochage (voir TCO), on remet à zero le détecteur optique, on active les fonctions liées aux capteurs optiques

PROG 2 AUT E1 AUTON BOARD 0 TRACK 3 STA ONTRACK ACT GPIO 0 0

PROG 0 AUT D6 AUTOFF BOARD 0 TRACK 3 STA ONTRACK ACT AUTON DA

PROG 0 AUT D7 AUTOFF BOARD 0 TRACK 3 STA ONTRACK ACT AUTON DB

PROG 0 AUT D8 AUTOFF BOARD 0 TRACK 3 STA ONTRACK ACT AUTON DC

PROG 0 AUT D9 AUTOFF BOARD 0 TRACK 3 STA ONTRACK ACT AUTON DD

// Lorsque la locomotive passe sur le capteur optique après le système de dételage, on inverse le sens de la locomotive et des aiguillages et on stoppe le dételage (voir TCO)

PROG 0 AUT DA AUTOFF BOARD 2 GPIO 0 1 ACT TRACK 30 SPEED 6 FORW INERTIA 27

// On désactive la fonction de remise à zero du capteur optique

PROG 2 AUT E2 AUTON BOARD 2 GPIO 0 1 ACT AUTOFF E1

// Le deuxième passage devant le capteur optique stoppe le dételage (voir TCO)

// Lorsque la locomotive passe sur l'autre capteur optique, on remet les aiguillages correctement (voir TCO) et on inverse le sens de la locomotive

PROG 0 AUT DB AUTOFF BOARD 2 GPIO 0 3 ACT TRACK 30 SPEED 6 BACK INERTIA 27

// Dès que le premier wagon passe devant le capteur optique après le dételage on retourne vers la gare centrale

PROG 0 AUT DC AUTOFF BOARD 2 GPIO 0 5 ACT TRACK 30 SPEED 8 FORW INERTIA 27

// ACtivation du TIMER 1 pour retour vers le mode normal de la carte

PROG 0 AUT DD AUTOFF  BOARD 2 GPIO 0 5 ACT TIMER 1 8

/////////////////////////////////////////////////////////

// CARTE 1 pour les 4 voies de la crémaillère I0,J1,K2,L3

// TIMER 1 => Attente gare du haut avant décrochage locomotive

// TIMER 2 => démarrage du train à crémaillère vers les voies centrales pour retournement

// TIMER 3 => mode normal de la voie

// TIMER 4 => démarrage retour arrière locomotive

// TIMER 5 => démarrage retour avant pour accrochage locomotive

// TIMER 6 => Retour vers la gare centrale

/////////////////////////////////////////////////////////

// Passage en mode automatique

PROG 1 AUT A0 AUTON BOARD 1 GPIO 3 1 ACT AUTOMATIC

// Eteint la led rouge du bouton mode manuel

PROG 1 AUT A1 AUTON BOARD 1 GPIO 3 1 ACT LPO 3 0

// Allume la led du bouton démarrer (Clignotement)

PROG 1 AUT A2 AUTOFF BOARD 1 GPIO 1 1 ACT LPO 2 2

// Lance la séquence automatique programmée

// DESCRIPTION DE LA SEQUENCE

// -------------------------

// Sur appui bouton démarrer, le train à crémaillère quitte la gare principale pour se rendre à la gare du haut. Après un temps d'attente de 5 unités, la locomotive se decroche et se positionne à l'arrière du train pour revenir vers la gare centrale. A l'arrivée à la gare centrale, après un temps d'attente de 5 unités, le train à crémaillère va sur la voie interne pour un décrochage de la locomotive et positionnement à l'avant du train en voie cachée avant de revenir en gare centrale puis remonter directement à la gare du haut

// Arme TIMER 3 pour déclenchement mode normal automatique

PROG 1 AUT A3 AUTOFF BOARD 1 GPIO 1 1 ACT TIMER 3 1

// Positionne correctement les aiguillages du haut

PROG 1 AUT A4 AUTOFF BOARD 1 TIMER 3 ACT LPO 4 0

// Demarre le train (qui doit être quelque part sur la voie I0 ou L3 (gare) )

PROG 1 AUT A5 AUTOFF BOARD 1 TIMER 3 ACT TRACK 31 SPEED 8 FORW INERTIA 27

// Active détection arrivée en gare du haut et activation TIMER

PROG 1 AUT A7 AUTOFF BOARD 1 TIMER 3 ACT AUTON D2

PROG 1 AUT A8 AUTOFF BOARD 1 TIMER 3 ACT AUTON D3

// Désactive arret du train en gare centrale

PROG 1 AUT A9 AUTOFF BOARD 1 TIMER 3 ACT AUTOFF D1

// Désactive armement TIMER 2 pour décrochage bas

PROG 1 AUT A9 AUTOFF BOARD 1 TIMER 3 ACT AUTOFF D4

PROG 1 AUT AA AUTOFF BOARD 1 TIMER 3 ACT AUTOFF D5

// Désactive Détection arrivée en gare du haut et activation TIMER

PROG 1 AUT AB AUTOFF BOARD 1 TIMER 1 ACT AUTOFF D2

PROG 1 AUT AC AUTOFF BOARD 1 TIMER 1 ACT AUTOFF D3

// Démarrage pour décrochage wagon

PROG 1 AUT AD AUTOFF BOARD 1 TIMER 1 ACT TRACK 31 SPEED 4 FORW INERTIA 40

// Arme TIMER 4 pour retour arrière locomotive

PROG 1 AUT AE AUTOFF BOARD 1 TIMER 1 ACT TIMER 4 2

// retour arrière locomotive sur déclenchement TIMER 4

PROG 1 AUT AF AUTOFF BOARD 1 TIMER 4 ACT TRACK 31 SPEED 5  BACK INERTIA 10

// changement aiguillage sur déclenchement TIMER 4

PROG 1 AUT AG AUTOFF BOARD 1 TIMER 4 ACT LPO 4 1

// Arme TIMER 5 pour retour avant locomotive

PROG 1 AUT AH AUTOFF BOARD 1 TIMER 4 ACT TIMER 5 2

//Retour avant de la locomotive au déclenchement TIMER 5

PROG 1 AUT AI AUTOFF BOARD 1 TIMER 5 ACT TRACK 31 SPEED 4 FORW INERTIA 27

// changement aiguillage sur déclenchement TIMER 5

PROG 1 AUT AJ AUTOFF BOARD 1 TIMER 5 ACT LPO 4 0

// Arme TIMER 6 pour retour final locomotive

PROG 1 AUT AK AUTOFF BOARD 1 TIMER 5 ACT TIMER 6 2

// Retour du train vers la gare centrale (voie I0, J1, K2 et L3)

PROG 1 AUT AL AUTOFF BOARD 1 TIMER 6 ACT TRACK 31 SPEED 5  BACK INERTIA 15

// Active arret du train en gare centrale

PROG 1 AUT AM AUTOFF BOARD 1 TIMER 6 ACT AUTON D1

// Active armement TIMER 2 pour décrochage bas

PROG 1 AUT AN AUTOFF BOARD 1 TIMER 6 ACT AUTON D4

PROG 1 AUT AO AUTOFF BOARD 1 TIMER 6 ACT AUTON D5

////////////////////////////////////////////////////////////////////////

// Démarre le train qui passe sur la CARTE 0

PROG 1 AUT AP AUTOFF BOARD 1 TIMER 2 ACT TRACK 31 SPEED 7  BACK INERTIA 15

// Désactive Arret du train lors de l'arrivée en gare centrale

PROG 1 AUT AQ AUTOFF BOARD 1 TIMER 2 ACT AUTOFF D1

// Arme TIMER 3 pour remise en mode normal de la voie

PROG 1 AUT AR AUTOFF BOARD 1 TIMER 2 ACT TIMER 3 2

////////////////////////////////////////////////////////////////////////

// Commande pouvant être désactivée

// --------------------------------

// Arret du train lors de l'arrivée en gare centrale (voie I0, J1, K2 et L3)

PROG 1 AUT D1 AUTOFF BOARD 1 TRACK 0 STA ONTRACK ACT TRACK 31 SPEED 0  BACK INERTIA 100

// Détection du train sur la voie K, arret des voies I0, J1, K2 et L3

PROG 1 AUT D2 AUTOFF BOARD 1 TRACK 2 STA ONTRACK ACT TRACK 31 SPEED 0 FORW INERTIA 27

// Arme TIMER 1 pour attente en gare du haut pendant 5 unités

PROG 1 AUT D3 AUTOFF BOARD 1 TRACK 2 STA ONTRACK ACT TIMER 1 5

// Arme TIMER 2 pour séquence de décrochage vers la partie cachée

PROG 1 AUT D4 AUTOFF BOARD 1 TRACK 0 STA ONTRACK ACT TIMER 2 5

// Désactive l'armement du TIMER 2

PROG 1 AUT D5 AUTOFF BOARD 1 TRACK 0 STA ONTRACK ACT AUTOFF D4

///////////////////////////////////////////////////////////////////////

// CARTE 2 pour la voie 0 du tramway et les 2 voies temporaire Faller AMS

// TIMER 1 => Temps d'attente devant les gares principales et intermédiaires à l'aller

// TIMER 2 => Temps d'attente à la gare devant la grue

// TIMER 3 => Temps d'attente à la gare intermédiaire au retour

///////////////////////////////////////////////////////////////////////

// Passage en mode automatique

PROG 2 AUT A0 AUTON BOARD 2 GPIO 3 1 ACT AUTOMATIC

// Eteint la led rouge du bouton mode manuel

PROG 2 AUT A1 AUTON BOARD 2 GPIO 3 1 ACT LPO 3 0

// Allume la led du bouton démarrer (clignotement)

PROG 2 AUT A2 AUTOFF BOARD 2 GPIO 1 1 ACT LPO 2 2

// Lance la séquence automatique programmée

// DESCRIPTION DE LA SEQUENCE

// --------------------------

// Sur appui bouton démarrer, et sous réserve que le Tramway ait bien été positionné devant la gare centrale, il va venir se positionner devant la gare du tramway à droite de la gare centrale, puis suivre un cycle d'aller/retour régulier avec un arret à l'aller et au retour à la gare intermédiaire. Chaque arret est programmé sur un TIMER de 5 unités

// Démarre le tramway pour le ramener sur la droite, le tramway doit être devant la gare de train

PROG 2 AUT A3 AUTOFF BOARD 2 GPIO 1 1 ACT TRACK 0 SPEED 5 BACK INERTIA 27

// Réinitialise GPIO 2 pour comptage position tramway

PROG 2 AUT A4 AUTOFF BOARD 2 GPIO 1 1 ACT GPIO 2 0

// Démarre les voies de voiture (temporaire)

PROG 2 AUT A5 AUTOFF BOARD 2 GPIO 1 1 ACT TRACK 1 SPEED 5 FORW INERTIA 1

PROG 2 AUT A6 AUTOFF BOARD 2 GPIO 1 1 ACT TRACK 2 SPEED 6 FORW INERTIA 1

// Allume led TCO droite M0

PROG 2 AUT A7 AUTOFF BOARD 2 GPIO 2 2 ACT LPO 5 1

// Stop le tramway

PROG 2 AUT A8 AUTOFF BOARD 2 GPIO 2 2 ACT TRACK 0 SPEED 0 BACK INERTIA 27

// Arme TIMER 1 d'attente en gare

PROG 2 AUT A9 AUTOFF BOARD 2 GPIO 2 2 ACT TIMER 1 5

// Redémarre le tramway sur déclenchement TIMER 1

PROG 2 AUT AA AUTOFF BOARD 2 TIMER 1 ACT TRACK 0 SPEED 5 FORW INERTIA 27

// Eteint led TCO droite M0

PROG 2 AUT AB AUTOFF BOARD 2 GPIO 2 4 ACT LPO 5 0

// Gare intermédiaire

PROG 2 AUT AC AUTOFF BOARD 2 GPIO 2 6 ACT TRACK 0 SPEED 0 FORW INERTIA 27

// Arme TIMER 1 d'attente en gare intermédiaire

PROG 2 AUT AD AUTOFF BOARD 2 GPIO 2 6 ACT TIMER 1 5

// ferme les barrieres

PROG 2 AUT AE AUTOFF BOARD 2 GPIO 2 8 ACT LPO 0 0

PROG 2 AUT AF AUTOFF BOARD 2 GPIO 2 8 ACT LPO 1 1

PROG 2 AUT AG AUTOFF BOARD 2 TIMER 2 ACT LPO 0 0

PROG 2 AUT AH AUTOFF BOARD 2 TIMER 2 ACT LPO 1 1

// ouvre les barrieres

PROG 2 AUT AI AUTOFF BOARD 2 GPIO 2 12 ACT LPO 0 1

PROG 2 AUT AJ AUTOFF BOARD 2 GPIO 2 12 ACT LPO 1 0

PROG 2 AUT AK AUTOFF BOARD 2 GPIO 2 20 ACT LPO 0 1

PROG 2 AUT AL AUTOFF BOARD 2 GPIO 2 20 ACT LPO 1 0

PROG 2 AUT AM AUTOFF BOARD 2 GPIO 2 2 ACT LPO 0 1

PROG 2 AUT AN AUTOFF BOARD 2 GPIO 2 2 ACT LPO 1 0

// Stop le tramway

PROG 2 AUT AO AUTOFF BOARD 2 GPIO 2 14 ACT TRACK 0 SPEED 0 FORW INERTIA 27

// Allume led TCO gauche M0

PROG 2 AUT AP AUTOFF BOARD 2 GPIO 2 14 ACT LPO 4 1

// Arme TIMER 2 d'attente en gare

PROG 2 AUT AQ AUTOFF BOARD 2 GPIO 2 14 ACT TIMER 2 5

// Redémarre le tramway sur déclenchement TIMER 2

PROG 2 AUT AR AUTOFF BOARD 2 TIMER 2 ACT TRACK 0 SPEED 5 BACK INERTIA 27

// Eteint led TCO gauche M0

PROG 2 AUT AS AUTOFF BOARD 2 GPIO 2 16 ACT LPO 4 0

// Gare intermédiaire retour

PROG 2 AUT AT AUTOFF BOARD 2 GPIO 2 22 ACT TRACK 0 SPEED 0 FORW INERTIA 27

// Arme TIMER 3 d'attente en gare intermédiaire

PROG 2 AUT AU AUTOFF BOARD 2 GPIO 2 22 ACT TIMER 3 5

// Redémarre le tramway sur déclenchement TIMER 3

PROG 2 AUT AV AUTOFF BOARD 2 TIMER 3 ACT TRACK 0 SPEED 5 BACK INERTIA 27

// Réinitialise GPIO 2 pour nouveau cycle aller-retour

PROG 2 AUT AW AUTOFF BOARD 2 GPIO 2 24 ACT GPIO 2 0

///////////////////////////////////////////////////////////////////////

// CARTE 3 AIGUILLAGES

///////////////////////////////////////////////////////////////////////

PROG 3 AUT M1 AUTON BOARD 3 GPIO 0 0 ACT GPIO 0 0

PROG 3 AUT M2 AUTON BOARD 3 GPIO 1 0 ACT GPIO 1 0

PROG 3 AUT M3 AUTON BOARD 3 GPIO 2 0 ACT GPIO 2 0

PROG 3 AUT M4 AUTON BOARD 3 GPIO 3 0 ACT GPIO 3 0

//============================================================

// GESTION DES COMMANDES DU TCO EN MODE AUTOMATIQUE

//============================================================

// Pilotage automatique décrochage circuit caché

PROG 31 AUT T0 AUTON BOARD 0 GPIO 3 0 ACT AUTOFF T2

PROG 31 AUT T1 AUTON BOARD 0 TIMER 1 ACT AUTOFF T2

PROG 31 AUT T2 AUTON BOARD 0 TRACK 3 STA ONTRACK ACT LPO 4 1

PROG 31 AUT T3 AUTON BOARD 1 TIMER 2 ACT AUTON T2

PROG 31 AUT T4 AUTON BOARD 2 GPIO 0 1 ACT AUTOFF T2

PROG 31 AUT T5 AUTON BOARD 2 GPIO 0 1 ACT LPO 4 0

// Commande automatique aiguillage E0 / (E0) H1 et E0 / G2

PROG 1 AUT T0 AUTON BOARD 0 GPIO 3 0 ACT AUTOFF T2

PROG 1 AUT T1 AUTON BOARD 0 TIMER 1 ACT AUTOFF T2

PROG 1 AUT T2 AUTON BOARD 2 GPIO 0 1 ACT LPO 0 1

PROG 1 AUT T3 AUTON BOARD 1 TIMER 2 ACT AUTON T2

PROG 1 AUT T4 AUTON BOARD 2 GPIO 0 4 ACT AUTOFF T2

PROG 1 AUT T5 AUTON BOARD 2 GPIO 0 3 ACT LPO 0 0

// Pilotage automatique décrochage gare crémaillère haut

PROG 0 AUT T1 AUTON BOARD 1 TIMER 1 ACT LPO 4 1

PROG 0 AUT T2 AUTON BOARD 1 TIMER 4 ACT LPO 4 0

PROG 3 AUT T1 AUTON BOARD 1 TIMER 2 ACT LPO 1 0

PROG 3 AUT T2 AUTON BOARD 1 TIMER 2 ACT LPO 0 1

// Commande automatique B1 / CROSS

PROG 3 AUT T3 AUTON BOARD 31 GPIO 1 1 ACT LPO 2 1

PROG 3 AUT T4 AUTON BOARD 31 GPIO 1 1 ACT LPO 3 0

// Commande automatique D3 / H3 et C2 / G2

PROG 3 AUT T5 AUTON BOARD 31 GPIO 1 1 ACT LPO 5 0

PROG 3 AUT T6 AUTON BOARD 0 GPIO 1 1 ACT LPO 5 0

// Commande automatique CROSS

PROG 3 AUT T7 AUTON BOARD 0 TIMER 1 ACT LPO 1 1

PROG 3 AUT T8 AUTON BOARD 0 TIMER 1 ACT LPO 0 0

PROG 3 AUT T9 AUTON BOARD 1 TIMER 2 ACT LPO 1 0

PROG 3 AUT TA AUTON BOARD 1 TIMER 2 ACT LPO 0 1

//============================================================

// GESTION DES COMMANDES DU TCO EN MODE MANUEL

//============================================================

// Pilotage manuel décrochage circuit caché

PROG 31 AUT T6 AUTON BOARD 3 GPIO 3 1 ACT LPO 4 1

PROG 31 AUT T7 AUTON BOARD 3 GPIO 3 0 ACT LPO 4 0

// Remise à zero GPIO

PROG 31 AUT T8 AUTON BOARD 31 GPIO 0 0 ACT GPIO 2 0

PROG 31 AUT T9 AUTON BOARD 31 GPIO 2 0 ACT GPIO 0 0

// Pilotage manuel décrochage gare crémaillère haut

PROG 0 AUT T3 AUTON BOARD 3 GPIO 2 1 ACT LPO 4 1

PROG 0 AUT T4 AUTON BOARD 3 GPIO 2 0 ACT LPO 4 0

// Commande manuel D3 / H3 et C2 / G2

PROG 0 AUT T5 AUTON BOARD 0 GPIO 0 0 ACT GPIO 0 0

PROG 0 AUT T6 AUTON BOARD 0 GPIO 2 0 ACT GPIO 2 0

// Commande manuel aiguillage E0 / (E0) H1 et E0 / G2

PROG 1 AUT T6 AUTON BOARD 1 GPIO 2 1 ACT LPO 0 1

PROG 1 AUT T7 AUTON BOARD 1 GPIO 0 1 ACT LPO 0 0

PROG 1 AUT T8 AUTON BOARD 1 GPIO 2 0 ACT GPIO 2 0

PROG 1 AUT T9 AUTON BOARD 1 GPIO 0 0 ACT GPIO 0 0

// Commande manuel aiguillage I0 / J1 , K2

PROG 1 AUT TA AUTON BOARD 3 GPIO 0 1 ACT LPO 4 1

PROG 1 AUT TB AUTON BOARD 3 GPIO 1 1 ACT LPO 4 0

// Commande manuel CROSS

PROG 3 AUT TB AUTON BOARD 31 GPIO 0 1 ACT LPO 1 1

PROG 3 AUT TC AUTON BOARD 31 GPIO 2 1 ACT LPO 0 1

PROG 3 AUT TD AUTON BOARD 31 GPIO 2 1 ACT LPO 1 0

PROG 3 AUT TE AUTON BOARD 31 GPIO 0 1 ACT LPO 0 0

// Commande manuel D3 / H3 et C2 / G2

PROG 3 AUT TF AUTON BOARD 0 GPIO 0 1 ACT LPO 5 1

PROG 3 AUT TG AUTON BOARD 0 GPIO 2 1 ACT LPO 5 0

// Commande manuel B1 / CROSS

PROG 3 AUT TH AUTON BOARD 31 GPIO 0 2 ACT LPO 2 1

PROG 3 AUT TI AUTON BOARD 31 GPIO 2 2 ACT LPO 3 1

PROG 3 AUT TJ AUTON BOARD 31 GPIO 2 2 ACT LPO 2 0

PROG 3 AUT TK AUTON BOARD 31 GPIO 0 2 ACT LPO 3 0

//============================================================

// REMISE EN ROUTE DES AUTOMATISMES

//============================================================

RUNALL

# Plants & Hardware Systems

## "Entry-level" ≲ 300 €

- PIKO SmartControl light (55017) — 2 A control unit + housing/handle, DCC, scalable via LocoNet (boosters). It can be found around ~€140–200 depending on the store. Coastal DCCConrad Electronic

- Digitrax Zephyr Express (DCS52) — ~3 A, DCC, USB, spikes/acc. and backroom control unit possible via Digitrax modules. Typical UK prices ~£190–£225. Geizhals.deRails of Sheffield

- NCE Power Cab — 2 Amp handle/center, simple and reliable, scalable to more powerful systems (SB5, etc.). Often ~£190–£210. Rails of Sheffielddcctrainautomation.co.uk

- Märklin Mobile Station 2 (60657) — Multi-protocol controller (mfx/motorola/DCC) for Märklin/Trix networks; requires track box. Price ~£80–£120 depending on kits. PriceRunnermaerklin.de

- Roco/Fleischmann z21 start — affordable DCC core (often in a box), Wi-Fi and app activable via upgrade; good gateway to the z21 ecosystem. In-Gauge Ltd Dapol Spares & Repairs

For whom? Small/medium networks, 1–3 simultaneous trains, first scanning.

## "Mid-range" ~ €300–600

- Roco z21 (black, 10820) — LAN/Wi-Fi/app, RailCom, smartphone control, large ecosystem (WLAN-Maus, MultiMaus, apps, iTrain/TrainController, etc.). Typically ~£350–£460. piko-shop.deDCCconcepts

- Gaugemaster / MRC Prodigy Advance² (DCC02) — 3.5 A control panel, club thinking, extension by wired/wireless joysticks and modules. ~£400–£495 depending on offers. gaugemasterretail.comCoastal DCC

- Lenz LZV200 (or Set101) — XpressNet/RailCom reference, very rugged; LZV200 single UVP €344.50; Set101 UVP 399–589 € depending on the version (wired/wireless). Lenz Elektronik+1

- ESU CabControl (50310) — built-in 7 A control panel + wireless touch control, Wi-Fi/LAN, ECoSlink (ESU boosters). Typically positioned ~€500–600. esu.euovrtrains.com

- Uhlenbrock Intellibox 2neo (65150) — multi-bus (LocoNet, XpressNet, s88...), modern "all-in-one" approach. dcctrainautomation.co.uk

For whom? Scalable HO/OO networks (3–6 trains), need for Wi-Fi/app, feedback and convenient PC automation.

## "High-end / club / large network" ≥ 600 €

- ESU ECoS 2.1 / 2.5 (50210/50220) — 7" touchscreen, 6–7 A, DCC+mfx/M4, RailCom, ECoSlink bus; popular in Europe. Price often ~£590–£700 (€≈680–800). Coastal DCCmodellbahnshop-lippe.com

- Märklin Central Station 3 (CS3, 60226) — multi-protocol (mfx/mfx+/DCC/Motorola) with display, TCO, built-in automation. Observed prices ~570 €+. Idealo

- ZIMO MX10 / MX10EC — extreme power (up to 20 A), CAN, advanced RailCom; clearly "pro/giant layout". Price ~£1,050 in the UK / ~US$1,559. Coastal DCCtrainli.com

- Massoth DiMAX 1210Z (Garden/G) — up to 12 A, designed for outdoor G scale. In UK stores ~£915. Garden Railway Specialists

For whom? Large networks, clubs, high intensity (many trains/lighting/sound), advanced integration.

## PC-centric solutions & software (for automation)

- iTrain (Lite → Pro) — €119 / €209 / €269 / €349 depending on the edition; TCO, routes, retro, multi-interfaces. berros.eu

- TrainController (Railroad & Co.) Bronze/Silver/Gold — widely used in advanced automation; Gold sells for around ~€650 (varies by country/tax). freiwald.comRMweb

- Rocrail & JMRI — free/open-source, very capable (TCO, auto, programming). majorgeeks.comjmri.org

- Hornby HM7000 (BLE) — control of locos via Bluetooth decoders + app, without a "big" classic DCC control unit; interesting in OO/GB. uk.hornby.comGoogle Play

- Hornby eLink + RailMaster — historical PC interface (fluctuating availability these days). uk.hornby.comnewmodellersshop.co.uk

All these software programs communicate with the listed control panels via LAN/USB (z21, ECoS, Lenz, Digitrax, NCE, DCC-EX, etc.). Check the exact compatibility on the hardware driver side. jmri.org

## DIY/open-source (very economical)

- DCC-EX (EX-CommandStation / CSB1) — open-source Arduino control panel (Wi-Fi possible), ready-to-use from ~€115–€120 in Europe; extremely scalable. dcc-ex.com+1

# Market Estimate (units/year)

There are no  precise public statistics by model or manufacturer for control centres. However, we can give a  reasonable order of magnitude by triangulating:

- Size of the model railroad market (all products combined) :p several sources of market research place global turnover around $1.16–1.22 billion in 2024–2025, with moderate growth. Market IntelligenceBusiness Research Insights

- Share of "control/electronics DCC" in hobbyist expenditure: conservative assumption 8–12% (the bulk of purchases remaining rolling stock, tracks, decoration). Unpublished hypothesis (I explain it for transparency).

- Average price of a central unit (all ranges combined, weighted by sales): ~€400–500 (in view of the prices observed above). (Based on detailed prices and in-store readings.)

Range calculation

- Low bound: $1.16 billion × 8% ≈ $93 million; $93 million / $500 ≈ ~185,000 units/year.

- High Bound: $1.79 billion × 12% ≈ $215 million; $215 million / $450 ≈ ~480,000 units/year. Market Intelligence

👉 Plausible order of magnitude: ~200,000 to 500,000 power plants sold/year worldwide.
In addition, there are tens of thousands of paid software licenses (iTrain, TrainController) and a large volume of users of free solutions (JMRI/Rocrail), but the exact figures are not published. berros.eufreiwald.comjmri.org

Note : depending on the region (Germany/Austria/Switzerland, United Kingdom, United States, Japan), ecosystem preferences vary (Märklin/mfx on the 3-rail side, z21/ECoS/Lenz on the DCC 2-rail side), which influences the distribution of sales by brand. The above orders of magnitude remain global.

## Quick advice to choose

- You get started : PIKO SmartControl light, NCE Power Cab, Digitrax Zephyr, z21 start.

- Scalable HO network with smartphone & PC : Roco z21 (black), Lenz LZV200 (with iTrain), ESU CabControl.

- Club / large network / garden : ESU ECoS, Märklin CS3 (3-rail), ZIMO MX10, Massoth 1210Z.

### 🔎 Key features of this DIY interface

- PIC18F4585/4680 microcontroller with DCC NMRA and analog PWM management

- 4 card-controlled channel sections (DCC or analog, but not mixed on the same board)

- Integrated vehicle presence detection (diodes + ADC)

- 6 low-power outputs (lighting, signals, small needle motors)

- 4 configurable GPIOs + 4 "track status" inputs (buttons, sensors, potentiometers, 7-segment displays)

- CAN bus (500 kBaud) to link multiple boards (up to 31 slaves + master)

- PC interface via RS232/USB FTDI + ASCII text command language

- Programmable automations (timers, train conditions, GPIO, etc.), stored in compressed EEPROM

- PC Qt (RailManager UI) application under development to simplify graphical programming

In other words: it is a distributed control system, hybrid rail + road (Faller AMS), with local automation + PC control.

### 💰 Positioning in relation to the existing market

If we compare it to the products in the overview I gave you earlier:

- Related features:

  - Multi-point control such  as a booster/modular control unit (e.g. Lenz LZV200 + modules, Roco z21 with extensions).

  - Standard NMRA DCC management  (such as Digitrax, NCE, ESU, Lenz).

  - Programmable local automation (rare at the entry-level; closer to high/mid-range solutions).

  - Built-in presence detection → advantage over the competition (often sold as separate modules).

- Differentiators:

  - AMS Dual Purpose Rail + Road Compatibility  → unique, niche yet innovative.

  - Reliable industrial CAN bus  → close to pro systems (ZIMO, Märklin CS3, Massoth).

  - Component prices (BOM page 23) ≈ ~€50–80 of raw components → low production cost, margin possible.

### 📊 Likely range of commercialization

- Entry-level (€<300 ): often limited to a basic control panel, without advanced automation. → This system offers more (multi-board, detection, automation).

- Mid-range (€300–600): where you can find z21 black (~€350–450), NCE Power Pro, Lenz LZV200 (~€400). These systems manage PCs, feedback signals, and several trains. → This interface aligns here in terms of power and modularity.

- High-end (€>600): typically ESU ECoS, Märklin CS3, ZIMO MX10, with touch screens, proprietary buses, high intensity. → This product is less "premium UI" oriented, so not in this category.

👉 So I would position this development in the mid-range range (~300–500 € per master kit complete with 1 card + PC interface).
Each additional slave card could sell for ~€100–150, which is still competitive with separate retro/detection modules (often €80–120 each from Lenz, Roco, Digitrax).

### 🚀 Market potential

- Niche "Faller AMS + HO mixed" (very differentiating, almost without competition).

- Wider market: model railways wanting simple automation without a PC → could compete with sets such as z21 + detection modules.

- Estimate: if we start on a global market of 200k–500k power plants/year (see my previous calculation), an innovative niche like this one could capture a few % → 1,000–5,000 realistic units/year if well distributed.

✅ In summary: This interface is positioned between a black z21 and a Lenz LZV200, so in the mid-range (300–500 €) with a competitive advantage on the integrated detection and AMS support. This is clearly above the entry-level but below the large premium power plants.

# 🛠️ Product Range Simulation

## "Starter Master" Pack

- Contents :

  - 1 Master card (PIC + USB/RS232 interface),

  - DCC/PWM management of 4 sections,

  - 6 auxiliary outputs,

  - 4 GPIO, built-in sensing,

  - RailManager UI (PC software).

- Positioning : entry into the ecosystem.

- Recommended retail price (RRP): €349

👉 Competitors: Roco z21 black (~€350–450), Lenz LZV200 (~€400).
👉 Advantage: Integrated detection, including simple automation.

## "Slave Expansion" card

- Contents :

  - Identical slave card (4 sections + detection + outputs),

  - Connected via CAN bus, automatically addressable.

- Recommended retail price : 129 €

👉 Comparison: Lenz/Roco retro modules = ~100–120 € for just the detection (without control).
👉 Here you offer detection + control in a single module → big perceived advantage.

## Advanced Automation Pack

- Contents :

  - RailManager UI (full version) with graphical editor (timers, conditions, scenarios),

  - Firmware enabling advanced distributed automation features (compressed EEPROM, macros).

- Recommended retail price : €99 (software license).

👉 Similar to iTrain "Lite" (119 €) or TrainController Bronze (≈150 €).
👉 Differentiator: embedded automation  (not just PC).

## "AMS Road Integration" package

- Contents :

  - Faller AMS specific road network adaptation kit (rails + dedicated sensors),

  - RailManager documentation/plug-ins.

- Recommended retail price : 79 €

👉 Unique product on the market (no direct equivalent).
👉 Can appeal to AMS collectors and clubs that mix rail + road.

# 📊 Summary range

- Master Starter : €349

- Slave Extension : 129 €

- Automation Advanced (software): €99

- AMS Road Kit : 79 €

➡️ Example "club" configuration: 1 Master (349 €) + 3 Slaves (3×129 €) + Automation (99 €) = ~835 € TTC→ consistent with an ECoS ESU (~700–800 €) but with more modularity.

# 🚀 Market strategy

- Core target : advanced modellers (clubs + demanding individuals).

- Advantage : Very competitive price per section compared to Lenz/Roco/ESU modular solutions.

- Differentiating niche : AMS Road Integration = unique element, attracts a small loyal community.

# 📑 Mini Business Plan – Rail Interface/AMS

For those who want to go further….

## Product

- Master Starter : Modular control unit with 4 sections, integrated sensing, basic automation, PC UI. (RRP: €349)

- Slave Extension : Extra 4-section module. (RRP: €129)

- Automation Advanced : Software license for scenarios and distributed automation. (RRP: 99 €)

- AMS Road Kit : Faller AMS road network integration. (RRP: €79)

⚡ Differentiators:

- Built-in detection by default (often modular option elsewhere).

- AMS rail + road compatible (unique).

- Distributed CAN (industrial reliability) architecture.

## Market and potential

- Global DCC Plants Market = 200k–500k units/year【Previous conversation】

- Niche penetration hypothesis: 0.5% (≈ 1,500–2,500 sales/year) in the 1st year, gradually increasing if clubs/forums are adopted.

- Likely distribution:

  - 1 Master each customer

  - 1.5 slaves/average customer

  - 40% customers take Automation license

  - 20% take AMS Road Kit

## Revenue model (per 1,000 customers)

| Product | Price (€) | Customers (%) | Volume (u) | Turnover (€) |
| --- | --- | --- | --- | --- |
| Master Starter | 349 | 100 % | 1 000 | 349 000 |
| Slave Extension | 129 | 150 % | 1 500 | 193 500 |
| Automation Advanced | 99 | 40 % | 400 | 39 600 |
| AMS Road Kit | 79 | 20 % | 200 | 15 800 |
| Total annual turnover |  |  |  | €598,000 |

## Cost structure

- BOM Master ≈ 70 € (PCB + components + housing + mounting).

- BOM Slave ≈ 50 €

- AMS software & kit → margin of almost 90%.

- Weighted average cost≈ 45% of the public price → gross margin 55%.

👉 On €598k turnover, gross margin ≈€329k.

## Fixed Expenses (Year 1)

- R&D finalization: 60 k€ (2 devs 6 months).

- Moulding of enclosures + CE certifications: 40 k€.

- Marketing (website, trade shows, forums, specialised pubs): 30 k€.

- After-sales service/support (freelance/part-time): 20 k€.

- Fixed year 1 ≈ 150 k€.

## Estimated ROI

- Annual gross margin : €329k

- Fixed year 1 : 150 k€

- Operating income : ≈ +€179k

➡️ ROI over 1 year: >100% (investment €150k, return ~€179k net from year 1 if 1,000 customers reached).
➡️ Break-even: ~450 customers (0.1–0.2% of the overall market).

## Competitive comparison (price per controlled section)

| System | Central Prize | Detection included? | Price per section (4) | Extensions (4 sections) |
| --- | --- | --- | --- | --- |
| This product | 349 € | ✅ Yes | 87 €/section | 129 € (32 €/section) |
| Roco z21 black | ~399 € | ❌ (detection in +) | 100 €/section | +100 €/4 detections |
| Lenz LZV200 | ~400 € | ❌ | 100 €/section | +110 €/8 detections |
| ESU ECoS 2.1 | ~700 € | ❌ | 175 €/section | + Modules |

👉 Clearly competitive  in €/section with detection included, especially once several slaves are added.

# 🎯 Conclusion

- This product is in the mid-range (€300–500) but offers more for the price (sensing, modularity, AMS).

- Differentiation strategy : combine rail + road, reduce total cost per controlled section.

- Viable business case: ROI possible in the first year with only 450–500 customers, which is realistic given the size of the market.