### 🏗️ General System Structure

- The system uses several boards (from board 0 to board 3, plus a special board "MASTER 31").

- Each board controls:

  - Up to 4 tracks (tracks A0 to D3 for board 31, E0 to H3 for board 0, etc.)

  - 6 low-power outputs (LPO): for LEDs, lights, switch controls, etc.

  - 4 GPIO inputs: for buttons, optical sensors, contacts, etc.

  - 4 "Track Status" inputs: for 7-segment displays, speed and inertia potentiometers.

### ⚙️ Operating Modes

#### ▶️ Automatic Mode

Commands like:

plaintext

CopyEdit

PROG <board> AUT <ID> AUTON/AUTOFF BOARD <n°> ... ACT ...

- Activates complex sequences with timing (TIMER), train detection (STA ONTRACK), rail actions (TRACK ... SPEED ...), LEDs (LPO ...), and I/O.

- Examples:

  - Launching a train based on a timer.

  - Controlling traffic lights (green/red) as trains pass.

  - Specific sequences for uncoupling wagons, level crossings, reversing locomotives, etc.

#### ⏸️ Manual Mode

Commands like:

plaintext

CopyEdit

PROG <board> AUT M<ID> AUTON BOARD <n°> ... ACT ...

- Resets GPIOs.

- Stops tracks (speed 0, inertia 0).

- Turns indicator LEDs on/off (e.g., start button LED, manual mode indicator).

### 🧠 Automation per Board

#### 🟩 MASTER 31 Board

- Controls tracks A0 to D3.

- Coordinates all other boards.

- Includes an optical control panel (TCO).

- Automations: loop sequences with station wait times, signal light control, train restart, synchronized timers (TIMER 1, TIMER 2, etc.).

#### 🟦 Board 0

- Controls tracks E0 to H3.

- Includes logic for uncoupling and coordination with board 1 (rack train).

- Advanced management of two trains coexisting on internal tracks.

#### 🟨 Board 1

- Controls tracks I0 to L3.

- Detailed sequence for fully automatic locomotive reversal.

- Uses six timers (TIMER 1 to TIMER 6).

- Tightly coupled with boards 0 and 2.

#### 🟧 Board 2

- Controls one tramway track (M0) and two temporary Faller AMS tracks.

- Extensive commands for level crossing lights, optical sensors, and regular back-and-forth sequences.

- Manages TCO LEDs to display tramway position.

#### 🟥 Board 3

- Dedicated board for manual/automatic switch control.

- Examples: toggling between main and passing track, dual-press manual commands.

### 📌 TCO (Optical Control Panel) Commands

- Enable manual or automatic control of events like:

  - Triggering red lights.

  - Switches when a train arrives or departs.

  - Activating or stopping uncoupling actions.

- Prefixes:

  - M<i>: manual mode.

  - A<i>: automatic.

  - T<i>: TCO command.

  - D<i>: deactivatable (can be temporarily removed from execution logic).

### ✅ Conclusion

This file represents a highly structured and powerful custom programming language designed to control a complex model train network, both automatically and manually. The commands:

- Are centered around interactions between the PC software, PIC18F microcontrollers, and physical elements (tracks, LEDs, buttons, sensors…).

- Are based on a master-slave architecture via CAN bus.

## Structure générale du système

- Le système utilise plusieurs cartes (de la carte 0 à la carte 3, plus une carte spéciale "MASTER 31").

- Chaque carte contrôle :

  - Jusqu’à 4 rails (voies A0 à D3 pour la carte 31, E0 à H3 pour carte 0, etc.)

  - 6 sorties basse puissance (LPO) : pour LEDs, feux, commandes d’aiguillages, etc.

  - 4 entrées GPIO : pour boutons, détecteurs optiques, contacts, etc.

  - 4 entrées "Track Status" : pour 7 segments, potentiomètres de vitesse et inertie.

## ⚙️ Modes d’opération

### ▶️ Mode automatique

Commandes de type :

plaintext

CopyEdit

PROG <carte> AUT <ID> AUTON/AUTOFF BOARD <n°> ... ACT ...

- Active des séquences complexes avec temporisation (TIMER), détection de train (STA ONTRACK), actions sur les rails (TRACK ... SPEED ...), LEDs (LPO ...), et entrées/sorties.

- Exemples :

  - Lancement d’un train selon minuterie.

  - Gestion du feu (vert/rouge) au passage du train.

  - Séquences spécifiques de décrochage de wagons, passages à niveau, retournement de locomotive, etc.

### ⏸️ Mode manuel

Commandes de type :

plaintext

CopyEdit

PROG <carte> AUT M<ID> AUTON BOARD <n°> ... ACT ...

- Remise à zéro des GPIOs.

- Mise à l’arrêt des voies (vitesse 0, inertie 0).

- Allumage ou extinction de LEDs indicatrices (ex. LED du bouton démarrer, mode manuel).

## 🧠 Automatismes par carte

### 🟩 Carte MASTER 31

- Gère les voies A0 à D3.

- Coordonne les autres cartes.

- Dispose d’un TCO (Tableau de Commande Optique).

- Automatismes : séquences de boucle avec attente en gare, gestion du feu, relance du train, synchronisation des temporisations (TIMER 1, TIMER 2, etc.).

### 🟦 Carte 0

- Gère voies E0 à H3.

- Intègre des logiques de dételage, coordination avec la carte 1 (train à crémaillère).

- Gestion poussée de la cohabitation avec un autre train sur voies internes.

### 🟨 Carte 1

- Voies I0 à L3.

- Détail d’une séquence complète de retournement automatique de locomotive.

- 6 temporisations (TIMER 1 à TIMER 6).

- Fortement couplée aux cartes 0 et 2.

### 🟧 Carte 2

- Gère une voie pour un tramway (voie M0) et 2 voies Faller AMS temporaires.

- Nombreuses commandes sur feux de passage à niveau, capteurs optiques, séquences aller-retour régulières.

- Manipule les LEDs du TCO pour visualiser la position du tramway.

### 🟥 Carte 3

- Carte dédiée au pilotage des aiguillages manuels/automatiques.

- Exemples : bascule entre voie principale et voie de dépassement, commandes double appui manuelles.

## 📌 Commandes TCO (Tableau de Commande Optique)

- Permettent de piloter manuellement ou automatiquement certains événements comme :

  - Le déclenchement du feu rouge.

  - Les aiguillages à l’arrivée ou au départ du train.

  - Le déclenchement ou arrêt des actions de dételage.

- Les préfixes :

  - M<i> : mode manuel.

  - A<i> : automatique.

  - T<i> : commande du TCO.

  - D<i> : désactivable (peut être supprimée temporairement dans la logique d'exécution).

## ✅ Conclusion

Ce fichier représente un langage de programmation personnalisé très structuré et puissant, conçu pour piloter un réseau de trains miniatures complexes de manière automatisée ou manuelle. Les commandes :

- Sont centrées autour des interactions entre le logiciel sur PC, les microcontrôleurs PIC18F, et les éléments physiques (rails, LEDs, boutons, capteurs…).

- Reposent sur une architecture maître-esclave via bus CAN.